## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engineering(CE601)/(4017) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the data collected during preliminary and detailed survey for an irrigation project? |  |
| b) | How measurement of rainfall is done by using Symons rain guage? |  |
| c) | Which are the factors affecting duty? How duty of a crop can be improved? |  |
| d) | Derive an expression for No-tension condition in Gravity Dam. |  |
| e) | Define Dead storage, live storage and gross storage. |  |
| f) | Write on type of gates used in spillway. |  |
| g) | What are the advantages and disadvantages of minor irrigation schemes? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat sketch explain weighing bucket type automatic rain guage to measure rainfall. |  |
| b) | The Gross-command area of an irrigation project is 1.6 lakh hectares, where 8000 hectares are unculturable. The area of Kharif crop is 65000 hectares and that of Rabi crop is 45000 hectares. The duty of Kharif crop is 3000 hectares/cumec and duty of Rabi crop is 4000 hectares/cumec. Find: (i) The design discharge of channel assuming 10% transmission loss (ii) Intensity of Kharif and Rabi. |  |
| c) | Derive a relationship between Delta, Duty and Base period of a crop. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat sketch explain self weight, water pressure and uplift pressure in Gravity Dam. |  |
| b) | Which are the factors to be considered in selecting Dam site? |  |
| c) | Explain hydraulic failure, seepage failure and structural failure in Earthen Dam. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which are the different types of losses in reservoir? |  |
| b) | What are the factors affecting reservoir losses? How to control reservoir losses? |  |
| c) | How Dams are classified? Write any three types of joints in Gravity Dam. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which are the different types of canal lining? Write any two advantages and disadvantages of canal lining. |  |
| b) | Explain typical cross-sections of canal in cutting, embankment, partial cutting and partial embankment. |  |
| c) | With the aid of neat sketch, explain i) Canal crossing over a drain ii) Drain crossing over a canal. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Investigation for an irrigation project |  |
| b) | Theissons polygon method |  |
| c) | Pheratic line in Earth dam |  |
| d) | Bandhara layout and component parts |  |

***********