## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Computer Engg.

Subject: Microprocessors & Applications(CO401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the following pins/signals of 8086: - WRii) MN/MXiii) Vcc |  |
| b) | Explain Based and Indexed addressing modes with an example of instruction. |  |
| c) | Explain the following 8086 instructions: - MOV BL, [SI] ii) ADD DX, BX iii) AND BX, 00FFH |  |
| d) | Explain the role of base pointer register in 8086. |  |
| e) | Explain PUSH operation. |  |
| f) | Write a short note on interfacing of ADC with 8086. |  |
| g) | Explain the role of instruction queue in 8086. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different functions performed by microprocessor. |  |
| b) | Explain the role of execution unit in 8086. |  |
| c) | Explain memory organisation of 8086. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an assembly language program to find average of two temperatures HI_TEMP and LO_TEMP and put the result in the memory location AV_TEMP. |  |
| b) | Predict the state of the six 8086 conditional flags after execution of each of the following instructions. Use the register contents shown below. Assume that all flags are reset before the instruction execute. - ADD CL, DH ii) ADD BL, CL iii) OR CX, BX |  |
| c) | Write an assemble language program to produce packed BCD from two ASCII characters. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain four types of CALL instructions giving example of each instruction. |  |
| b) | Explain the following assembler directives. State an example for each and explain. - DW ii) EQU iii) ENDS |  |
| c) | Compare macros and procedures. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the different types of 8086 interrupts. |  |
| b) | Explain interrupt response in 8086 with diagram. |  |
| c) | Explain the functions of the following signals in 8086: i) RESET ii) READYiii) BHE iv) ALE v) HOLD vi) M/IO |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give an overview of Pentium processor. |  |
| b) | Explain with diagram interfacing of 8086 with traffic light controller. |  |
| c) | Explain interfacing of 8086 with DAC giving diagram. |  |

***********