## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programm	e: Mechanical Engg.

Subject: Fluid Machinery(4040) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water 10mm in diameter issues with a velocity of 25m/s and impinges normally on a plate moving with a velocity of 10m/s. Find the force exerted by the jet on the plate. |  |
| b) | Define hydraulic efficiency of a turbine. |  |
| c) | Explain the term priming with respect to centrifugal pump. |  |
| d) | What do you understand by the term slip with respect to reciprocating pump. |  |
| e) | State applications of screw pump. |  |
| f) | What are the functions of shuttle valves? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain construction and working of a Pelton wheel turbine. |  |
| b) | What are the advantages and disadvantages of Francis turbine over a Pelton wheel turbine? |  |
| c) | Draw and explain the main characteristic curves of a Kaplan turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of a Francis turbine. |  |
| b) | Describe the multi-stage centrifugal pump with impellers in parallel. |  |
| c) | Explain the losses which occur when a centrifugal pump operates. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is cavitation? What are the effects of cavitation? Give the necessary precautions against cavitation in case of a centrifugal pump. |  |
| b) | What is an air vessel? What are the uses of air vessels? Show that work saved in a single acting reciprocating pump due to air vessel is 84.8%. |  |
| c) | Describe the working of a double acting reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a variable delivery pump. |  |
| b) | What is the function of a FRL unit in a pneumatic circuit? Explain a FRL unit with the help of a neat sketch. |  |
| c) | With the help of a neat sketch explain sequential hydraulic circuit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe a spool valve with the help of a neat sketch. |  |
| b) | What is the importance of a direction control valve in a pneumatic circuit? Explain the different types of actuation systems of direction control valve. |  |
| c) | How are reciprocating pumps classified? |  |

***********