## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Architecture Engg.

Subject: Building Construction-IV(AE402) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the three methods of connecting steel sections. |  |
| b) | State the basic difference between the technique of grouting and guniting. |  |
| c) | State the function of ‘Coba’ in the conventional waterproofing treatment over RCC roof slab. |  |
| d) | List three types of special mortars. |  |
| e) | List three types of joists used in timber floor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name any three rolled sections and explain them. |  |
| b) | Elaborate on any one method of connecting steelwork. |  |
| c) | State the general principles observed in design and construction of steel structures. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch & label a M.S. channel box section connected to a base plate fixed over concrete base. |  |
| b) | Draw a neat labelled sketch of I beam connected to hollow square column section. |  |
| c) | Draw a neat labelled sketch of I beam connected to I stanchion. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain any two preventive measures adapted to avoid dampness in a newly constructed building. | (3) |
| b) b) | Explain in brief three materials used for Damp proofing course. OR State the probable causes of dampness in a 40 years old residential apartment building. | (3) (3) |
| c) c) | Write a descriptive note on procedure of waterproofing adopted for a basement in damp soil. OR Draw a neat labelled diagram to explain waterproofing treatment for a underground RCC sump admeasuring 2m x 4m having 1.5m depth. | (6) (6) |
| Q.No.5. | Answer the following Questions: |  |
| a) a) | It is required to provide dado over a kitchen platform. Suggest suitable material and elaborate on procedure to execute the work with suitable sketches. OR Draw labelled sketches of various shapes of pointing (any 6 types) on a wall surface. | (6) (6) |
| b) | Explain the term ‘splicing’ with sketches. | (3) |
| c) | Draw a cross section through a brick jack arch floor. | (3) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: |  |
| a) a) | Elaborate on merits & demerits of Precast concrete floor. OR Write a descriptive note on ribbed floor construction. | (6) (6) |
| b) b) | Draw a neat labelled sketch through a triple joist timber floor. OR Draw a neat labelled cross section of RCC floor slab load bearing wall. Also show reinforcement details. | (6) (6) |

***********