## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(4032) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain perpendicular axis theorem with an example. |  |
| b) | Differentiate between thin and thick cylinders. |  |
| c) | State the assumption made in theory of pure bending. |  |
| d) | Define stress, strain and modulus of elasticity for steel rod subjected to axial tension. |  |
| e) | Define lateral strain and Poisson’s ratio. |  |
| f) | State the torsion equation and list the terms involved. |  |
| g) | State the relationship between Young’s modulus, Bulk modulus and Poisson’s ratio. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar consisting of two lengths, 180mm and 220mm having diameters 30mm and 20mm respectively is subjected to axial tensile force of 40KN at the ends. Find the stress in the two lengths and the total extension of the bar. E = 2 x105 N/mm2 |  |
| b) | A reinforced concrete column is 400mm x 400mm and is provided with 8 bars of 22mm diameter. Find the safe axial load on the column if the maximum stress in concrete in limited to 4N/mm2. Take Es=18 Ec. |  |
| c) | A steel rod is 25m long at a temperature of 20C. Find the free expansion of the rod if the temperature is raised to 65C. Find the temperature stress produced when the rod is allowed to expand by 6mm.  = 12 x 10-6 perC and E = 2 x 105 N/mm2 |  |
| Q.No.3. | Answer the following Question: | 2 x 6 = 12 |
| a) a) | At a certain point in a strained material the principal stresses are 100N/mm2 and 40N/mm2 both tensile. Find the normal, tangential and resultant stress across a plan through the point at 48 to the major principal plan using Mohr’s circle of stress. OR A certain material is subjected to principal stress of 80N/mm2 tensile and 50N/mm2 compressive. Find the resultant stress on plane making 64 with the major principal plane. Find also normal and tangential stresses on the plane. |  |
| b) | A load of 500N falls freely through a height of 150mm on to a collar attached to the end of a vertical rod of 50mm diameter and 2m long, the upper end of the rod being fixed to the ceiling. Calculate the maximum instantaneous extension of the bar. Also calculate the maximum stress in the bar. Take E = 2 x 105 N/mm2. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) a) | A simply supported beam of span 8m is subjected to three point loads 300N, 400N, 500N at every 2m from the left support. Draw the S.F.D. and B.M.D. OR A cantilever of span 6m is fixed at A and free at B. It is subjected to uniformly distributed load of 50N/m over the entire span and a point load of 100N at free end. Draw S.F.D and B.M.D. |  |
| b) | Find the moment of Inertia about horizontal centroidal axis and about bottommost fibre of ‘T’ section whose top flange is 100mm x 20mm, web thickness is 20mm and overall depth is 140mm. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel beam of hollow tube section of 60mm outer diameter and 50mm inner diameter is simply supported on a span of 4m. Find the maximum concentrated load the beam can carry at the middle of the span if the bending stress is not to exceed 120N/mm2 |  |
| b) | A masonry pillar 600mm x 600mm carries a point load of 1000KN acting on the centroidal axis of the section at an eccentricity of 80mm from longitudinal axis. Find the maximum and minimum stresses on the section. |  |
| c) | A 60 mm diameter shaft is subjected to a torque of 4KN-m. Find the maximum shear stress induced in the shaft. Also find the angle of twist per metre length of the shaft. C = 8 x 104 N/mm2 |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A shell 3.25m long, 1m in diameter is subjected to an internal pressure of 1N/mm2. If the thickness of the shell is 10mm, find the circumferential and longitudinal stresses. E = 2 x105 N/mm2 and Poisson’s ratio is 0.3. |  |
| b) | A strut 2.50m long is 60mm in diameter. One end of the strut is fixed and other end is hinged. Find the safe compressive load for the member using Euler’s formula, allowing factor of safety of 3.5. Take E = 2.1 x 105 N/mm2. |  |
| c) | A cantilever 3m long carries a point load of 100N at free end. If the slope at free end is 1, find the deflection at free end of the cantilever. |  |

***********