## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/MED

Subject: Analog Electronics(EN405) [Revied]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare the following ideal characteristics with typical characteristics of an opamp: i) Band width ii) input resistance iii) CMRR. |  |
| b) | Draw the block diagram of voltage series feedback and write expression for output voltage gain. |  |
| c) | For an opamp having R1 = 1, RF = 10K and Vin = 1Vdc, calculate output voltage when connected in i) inverting mode ii) non-inverting mode iii) differential mode with vin1 = 1V & vin2 = 2V. |  |
| d) | What are advantages of active filters over passive filters? |  |
| e) | With respect to Wein bridge oscillator circuit with opamp give the following: i) frequency of oscillation ii) gain for sustained oscillation iii) relation between R1 & RF for above condition. |  |
| f) | What are the advantages of IC voltage regulators? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) What are the requirements of a good differential amplifier circuit? ii) Give one advantage of differential amplifier. iii) Give a neat circuit diagram of dual input balanced output differential amplifier. |  |
| b) | Draw internal block diagram of an OPAMP and explain each block. |  |
| c) | Draw pin configuration of opamp IC741 and explain function of each pin. Draw circuit to nullify offset voltage for this IC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give reasons why open loop opamp configurations are unsuitable for linear applications. List types of feedback. Give any three advantages of negative feedback. |  |
| b) | Draw the circuit of an inverting opamp amplifier with three inputs. Explain how it can be used as i) summing amplifier ii) averaging amplifier. |  |
| c) | Explain with neat circuit diagram working of a basic integrator using opamp. Give input and output waveforms for sine and square waves as inputs. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of 1st order low pass Butterworth filter using opamp. Write the design steps for above filter. |  |
| b) | Design a Butterworth first order high pass filter with opamp, with cut off frequency 1KHz, pass band gain = 2 and C = 0.01f. Draw the required circuit. |  |
| c) | Draw the circuit diagram of peak detector using opamp. Explain with waveforms. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram for sinusoidal waveform generator using opamp. Explain the working with necessary waveforms. Write expression for resonant frequency. |  |
| b) | Draw the block diagram of 555 timer. What is the function of i) Reset ii) trigger iii) control voltage. |  |
| c) | Write advantages of adjustable voltage regulator over fixed voltage regulator. Draw the circuit of series voltage regulator using 7805. State the function of input and output capacitor. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | i) Give three features of 78XX series voltage regulator. ii) Draw the circuit diagram for low voltage regulator using IC723. |  |
| b) | Draw and explain working of voltage follower circuit. How is this circuit a special case of voltage series feedback? |  |

***********