## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-I(2005) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the type of vectors along with example. |  |
| b) | How to add finite number of forces? |  |
| c) | State the effect of force on a body. |  |
| d) | State the characteristics of couple. |  |
| e) | State Varignon’s theorem & explain. |  |
| f) | State & explain Lami’s theorem. |  |
| g) | Define angle of friction and angle of repose. |  |
| h) | Explain the V-T diagram & give the equation for acceleration, distance & retardation. |  |
| j) | With neat sketch locate the c.g. of a right circular cone. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In the Cartesian coordinate four forces are acting at the origin 3N, 5N, 2N and 4N. The force 3N is acting along +ve x-axis, 5N along +ve y-axis, 2N between –ve x & y-axis and 4N along –ve y-axis. Find out the resultant of the system of forces. |  |
| b) | Three forces of magnitude 20N, 30N and 40N are acting along the sides of an isosceles right angle triangle of side 2m in anticlockwise direction. Calculate the magnitude and direction of the forces and locate with respect to 90 corner. |  |
| c) | Find out the resultant of the system of forces shown in fig.1 & locate the resultant with respect to point ‘A’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a parallel force system there are four forces: 9N acting right side, 7N acting right side, 8N acting left side and 5N acting right side. The typical distance between two forces is 2m. Find the magnitude and direction of the resultant force and locate the resultant with respect to any force. |  |
| b) | Calculate the reaction offered by the walls of the channel in fig.2. |  |
| c) | Find out the force in the cable in fig.3. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam of length 10m is loaded as shown in fig.4. Determine the reactions at the support. |  |
| b) | Determine the reactions at the supports in fig.5. |  |
| c) | Calculate the force ‘P’ required to raise the block of weight 10N up the plane as shown in fig.6. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The weight of 100N is resting on the wedge as shown in the fig.7. The coefficient of friction is 0.3. Calculate the force to raise the block. |  |
| b) | Derive the Kinematic equations. |  |
| c) | A train travels between two stations 10 Km apart in 15 minutes. Assuming that its motion is with uniform acceleration for the first part of journey and with uniform retardation for the rest, show that the greatest velocity of the train is 80 Km/hr. | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A vehicle starts from station Á and comes to hault at station B 4.05 Km away on a straight road. At the beginning it accelerates uniformly for 40 seconds and attains maximum velocity ‘V’, with this velocity it travels for 4minutes. Then it retards uniformly and comes to rest at station B. The total time of journey is 5 minutes. Find the value of ‘V’. Also find acceleration and retardation. |  |
| b) | Calculate the c.g. of the fig.8. |  |
| c) | i) State the law of parallelogram of forces. ii) Define couple & give its unit. |  |