## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(EE612)/(5106) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Natural light (ii) Artificial light (ii) Supplementary light. |  |
| b) | Explain the significance of Polar curve. |  |
| c) | Define (i) Candle power (ii) M.S.C.P. (iii) M.H.C.P. |  |
| d) | State the factors affecting the selection of luminaires for indoor lighting. |  |
| e) | Give two points of distinction between Room Index and Cavity Index. |  |
| f) | What is flood lighting? Where it is used? |  |
| g) | Explain how street lighting is different from Indoor lighting. |  |
| h) | Give any three important features of staircase lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the difference between general diffused lighting system and mixed diffused lighting system. |  |
| b) | State the different types of lighting schemes & explain each of such schemes in brief with an example of each. |  |
| c) | What is refection of light? Explain with the help of neat sketches, different types of refections. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain what is maintenance factor. What are the factors on the basis of which maintenance factor for interior lighting design selected? |  |
| b) | i) State & explain Lambert Cosine Law. | (4) |
|  | ii) Define (i) Lumens (ii) Polar curve | (2) |
| c) | Two lamps are hung at a height of 9m from the floor level. The distance between the lamps is 2m. Lamp 1 is of 500 CP. If the illumination on the floor vertically below this lamp is 20 lux, find the candle power of lamp 2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the lumen or light flux method of lighting calculations. |  |
| b) | Explain what is i) Coefficient of utilization & ii) Maintenance factor. Discuss the factors on which they depend. |  |
| c) | A hall 30m x 15m with a ceiling height of 5m is to be provided with a general illumination of 120 lumens/m2. Taking coefficient of utilization of 0.5 & depreciation factor of 1.4, determine the number of fluorescent tubes required, and total wattage. Take luminous efficiency of fluorescent tube as 40 lumens per watt for 80 watt tubes. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the two general principles employed in design of street lighting installations. |  |
|  |  | ……2/- |
|  | -2- |  |
| b) | With the help of neat diagram, explain different arrangements for fixing of streetlight luminaries. Also give advantage of each. |  |
| c) | Streetlights are suspended at a height of 20m along the centre line at regular intervals of 24m. The Polar curve of the lamp with its reflector can be obtained by the following table. |  |
|  | C.P; 240; 270; 290; 260; 190; 130; 50; 15. Angle to vertical; 0; 10; 20; 30; 40; 50; 60; 70. |  |
|  | Calculate the intensity of illumination between two lamps along the centre line and at a distance of 6m, 12m and 18m from one lamp. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the steps involved in flood-lighting calculation. |  |
| b) | With reference to flood lighting: i) State the types of lamps used ii) Explain different locations & mounting of flood light projectors. |  |
| c) | i) State any three important features of Auditorium lighting. | (3) |
|  | ii) State any three important features of Hospital operation theater lighting. | (3) |

***********