## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(EE511)/(4108) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between feeder and distributor. (3 points) |  |
| b) | State the system voltage and permissible voltage drop in various parts of the distribution system for urban, sub urban and rural areas. |  |
| c) | State any three advantages and limitations of SWER system for rural electrification. |  |
| d) | State any three IE rules to be followed for light and fan wiring. |  |
| e) | Define earthing. State the necessity for earthing. |  |
| f) | List any six methods used for reducing earthing resistance. |  |
| g) | Classify and list the different types of faults occurring in distribution system. Show the diagrammatic/symbolic representation of each fault. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A single phase two wire distributor 2Km long supplies a load of 80A at 0.9 p.f. lagging at its mid point and 120A at 0.8 p.f. lagging at its far end. Both the power factors are referred to the voltage at the far end. The resistance and reactance per Km of the distributor are 0.05 and 0.1 respectively. If the voltage at the far end is maintained at 230V, calculate:- i) The voltage at the sending end (using the impedance method) ii) Phase angle between the voltages at the two ends. |  |
| b) | State and explain the factors on which selection of size of conductor for a distributor depends. |  |
| c) | i) Find the inductance per phase per Km of a 3, 3 wire distribution line having conductor of 1.2 cm diameter and when the conductors are placed at the corners of a triangle of sides 2.5m, 3.2m and 4m. ii) Find the loop inductance per Km of a single phase two wire distribution line having conductor of 1 cm diameter and placed parallel to each other with a distance of 1m between them. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A line is to be tapped off from an existing 3 phase, 3 wire, 11 KV line to supply to a 3 phase load of 150 HP at a distance of 400m. The load power factor is 0.85 lagging. Estimate the capacity (power and current) and other specifications of the transformer required. Also prepare bill of material required for erection of the line. Assume that there may be 25% increase in the load in future. |  |
| b) | List out any four desirable properties to be possessed by the following distribution line components: i) Line supports ii) line conductor iii) Insulators. | ……2/- |

|  | -2- |  |
| --- | --- | --- |
| c) | Estimate the requirement of main material for erecting a 3 phase, 3 wire, 11 KV overhead distribution line over a straight run of 1.5 Km. Out of the 1.5 Km length around 0.5 Km length is to be provided with cradle guarding. Consider average span of 100m and horizontal conductor configuration. Use 11m long rail poles and ACSR conductor 6/1 x 2.13 mm weighing approximately 150 Kg/Km. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain radial and ring type of distributor and state one advantage and limitation of each type. |  |
| b) | Explain the ABC system of rural electrification. State its advantages. |  |
| c) | With neat diagram explain plate earthing. |  |
| d) | Define fault. State percentage of occurrence of each fault on the distribution system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain wiring diagram for: - Godown wiring ii) For controlling one lamp, one fan and one socket using individual switches. |  |
| b) | State any six IE rules applicable for domestic wiring. |  |
| c) | A hall of 15 (l) x 6m (b) x 4.5m (h) needs to be wired up for providing 10 lamps of 100W each and 8 fans of 60W each. - Draw the layout/plan showing the location and distribution of lamps and fans. - Estimate and prepare the bill of material required for this wiring work. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and discuss the important considerations/IE rules regarding motor installations for industrial wiring system. |  |
| b) | Explain the following: i) Cable system ii) Bus way system (overhead). |  |
| c) | A small workshop of 15m (l) x 10m (b) x 5m (h) needs to be equipped with two induction motors of 415V, 3 phase, 15HP and 10HP at a distance of 2m and 3.5m respectively from the meter board/main switch. Draw the typical layout of wiring required and calculate the ratings of main switch, motor switch, fuses and cables required for the same. |  |

***********