## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(EE601)/(4115) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Electric Drive. Draw its block diagram. |  |
| b) | What are active load torques? Give one example. |  |
| c) | List methods of electrical braking applied to motors. |  |
| d) | State any three advantages of electric braking. |  |
| e) | What is a flywheel? Why it is used? |  |
| f) | Which type of 3-phase induction motor is used for cranes? Why? |  |
| g) | What is a selector switch? When it is used? |  |
| h) | Draw characteristics of speed control of three phase induction motor by voltage control. Hence state its disadvantage. |  |
| i) | State any three features of textile motors. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain joint speed torque characteristics of motor and load. |  |
| b) | With a neat diagram explain variation of load torques with angular displacement at shaft. |  |
| c) | What do you mean by stability of drive? Briefly explain steady state stability of drive system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of circuit diagram and characteristics explain rheostatic braking of d.c. shunt motor. |  |
| b) | With the help of circuit diagram and characteristics explain plugging of 3-phase induction motor. |  |
| c) | Briefly explain following methods of speed control of three phase induction motor: - Rotor resistance control - Change in applied voltage |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagrams for electric motor explain i) Short time duty ii) Intermittent periodic duty. |  |
| b) | Explain effect of load inertia on rating of motor. |  |
| c) | On the basis of heating, select a suitable motor for the following intermittent duty: i) constant load of 35 KW for 3 sec ii) constant load of 15 KW for 20 sec iii) constant load of 35 KW for 2 sec iv) constant load of 10 KW for 15 sec. Between the operating periods of (ii) & (iii) there is a period of rest for 37 seconds and after (iv) there in another rest period for 43 seconds. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of i) Push button ii) Overload relay. | ……2/- |
|  | -2- |  |
| b) | Draw a neat control diagram and explain operation of electric motor fed from two sources. |  |
| c) | With a neat power circuit and control circuit explain Forward-Reverse operation of induction motor. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Drive motor for refrigeration and air conditioning |  |
| b) | Time delay relay and its applications |  |
| c) | Slip power recovery scheme applied for induction motors |  |

***********