## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(EE403)/(4111) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any six methods of electric heating. |  |
| b) | List three desirable properties to be possessed by the resistance heating element. Give any three examples of material used for the same. |  |
| c) | Briefly explain the principle of direct arc heating. |  |
| d) | List any three advantages of coreless induction furnace. |  |
| e) | Briefly explain the concept and significance of arc stability with respect to arc welding. |  |
| f) | Define arc welding. Briefly explain its working principle. |  |
| g) | Define glare. Why is it undesirable in illumination? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is direct resistance heating? With neat diagram explain the construction and working of any one furnace based on direct resistance heating. |  |
| b) | Explain infrared heating. State its advantages & disadvantages (atleast three each). |  |
| c) | A 20KW, 230V, single phase resistance oven employes nickel-chromium wire for its heating element. If the element temperature is not to exceed 1200ºC, keeping temperature of the charge to 750ºC, calculate the diameter and length of the element wire. Assume the radiant efficiency to be 0.65 and emissivity as 0.9. Also the resistivity for nickel-chrome is 1.016 x 10-6 Ω-m |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain at least six advantages of electric heating over other types of heating. |  |
| b) | Explain principle of dielectric heating. State its any two advantages and applications. |  |
| c) | With neat circuit diagram explain the working principle of resistance welding. State its advantages and limitations (atleast three each). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the phenomena of arc blow in arc welding. List the methods of reducing arc blow. Also list its causes and effect on weld quality. |  |
| b) | Enlist various current controllers used in welding sources. Describe the construction and working of any one current controller. |  |
| c) | With the help of neat diagram explain any high frequency eddy current heating equipment. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various types of track electrification system used for electric traction. Briefly explain any two systems of your choice. |  |
| b) | Draw and explain the speed-time curve of electric trains for: (i) Main line (ii) Urban & (iii) Sub-urban services. | …….2/- |
|  | -2- |  |
| c) | With the help of neat diagram explain shunt transition method used for traction. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the working and construction of fluorescent tube light stating functions of its various components/parts. |  |
| b) | With neat diagram explain the construction & working of HPSV Lamp. State its major application. |  |
| c) | What is stroboscopic effect? With the help of example explain how it can be avoided. |  |

***********