## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT

Subject:Design of Structures-I(RCC)[CE501)/ Design of Structures-II(RCC) [4016]

[Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4)  Use of IS-456-2000 code is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out various loads to be considered in design of R.C. structures and state their combinations to be considered. |  |
| b) | What is partial safety factor? Stating its significance mention the values prescribed by the I.S. code for materials and loads. |  |
| c) | Prove that the critical N.A depth with Fe415 steel is 0.48d for an R.C. member, having effective depth ‘d’. |  |
| d) | What is under reinforced section? Explain why do you prefer such sections. |  |
| e) | Distinguish between one-way and two-way slabs. |  |
| f) | What is flanged beam? State necessary conditions to be fulfilled for flanged beam consideration. |  |
| g) | Sketch the reinforcement details for a column which offsets from larger to smaller cross section in the vertical direction. |  |
| h) | Sketch various cover blocks used for reinforced concrete sections. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An RC beam 230mm x 400mm (overall) is provided with 3 #16mm as tensile reinforcement. If M20 concrete and Fe415 steel are used, determine the safe working udl the beam can carry over the 4.5m simply supported span. |  |
| b) | Design a simply supported beam to carry an udl to 25KN/m under working condition over an effective span of 4m. Use M25 concrete and Fe415 steel. |  |
| c) | An R.C. beam 250mm x 500mm consists of 4#12mm. If M25 and Fe500 are concrete and steel, determine N.A depth, lever arm and moment of resistance of the beam. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported R.C beam 300mm x 500mm is provided with 5#12mm as tensile reinforcement, 2#12 bars are bentup at 45 near the support. The beam carries an udl of 30 KN/m at working condition over an effective span of 5m. Design the shear reinforcement if M20 concrete and Fe500 steel are used. |  |
| b) | Describe the concept of “Truss Analogy” for shear reinforcement in an R.C. beam. |  |
| c) | Calculate the development length Ld for the following in terms of diameter of steel reinforcement in tension. fy (N/mm2); M20; M25; M30. 415. 500. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the “Steel-Beam’’ theory and state its application. |  |
| b) | Design a R.C. beam, whose dimension is restricted to 230mm x 400mm to carry a service load of 40KN/m over an effective span of 6m. M25 and Fe 415 steel are used for beam. |  |
| c) | An RC beam 250mm x 450mm is provided with 2#16mm on compression side and 4#12mm on tension side of the beam. If the beam is simply supported over 5.5m span and a working load of 50KN/m is acting on the entire span, check the suitability of the section. M25 and Fe500 grades of concrete and steel are used for the beam. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | A cantilever chajja 600mm span, carries a superimposed load of 1.5KN/m2 and finishing load of 0.5KN/m2. With M20 concrete and Fe415 steel, design the chajja. | (4) |
| b) | Design three span, one-way continues slab of spans 3.10m, 3.25m and 3.10m respectively. The LL on slab is 2.5KN/m2 and finishing load may be taken as 1KN/m2. Use M20 concrete and Fe415 steel. OR | (8) |
| c) | Design a two-way slab for a hall measuring 4.5m x 5.5m to carry an imposed load of 2.0 KN/m2 and finishing load of 1 KN/m2. The thickness of support alround the slab is 230mm and slab is continuous an two adjacent supports and discontinuous on remaining two adjacent edges. Take M25 concrete and Fe415steel. | (8) |
| Q.No.6. | Answer any one of the following Question: | (12) |
| a) | Design a short column with suitable longitudinal and lateral reinforcement to carry an axial load of 750KN. The effective length of column is 3.0m, M25 and Fe415 steel are used for the column. Adopt - Circular section - Square section - Rectangular section | (4) (4) (4) |
| b) | Design a rectangular footing, to carry an axial column load of 600KN. The column is 250mm x 350mm and the bearing capacity of soil is 150KN/m2. Use M20 concrete and Fe500 steel. Apply necessary checks for the footing designed. |  |

***********