## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Electrical & Electronics(AT601) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain what is meant by ‘Ah’ rating of a battery. |  |
| b) | Explain in brief how a maintenance free battery is different from a non-maintenance free battery with regards to its construction. |  |
| c) | What is the difference between a starting system cable and an ignition system cable? |  |
| d) | Why should the starting motor be disengaged from the engine flywheel ring gear after the engine starts? |  |
| e) | What is the function of the capacitor in the ignition system? |  |
| f) | List out the functions of the charging system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction of a lead-acid battery. |  |
| b) | State and explain the different ratings of a lead acid battery, |  |
| c) | Explain the causes of failure of a lead acid battery. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the trickle and booster charging of batteries. |  |
| b) | Explain electronic ignition system. |  |
| c) | Explain conventional ignition system with a diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction of sealed beam lamp. |  |
| b) | With a neat sketch explain the construction and operation of a fuel gauge. |  |
| c) | With a neat sketch explain the operation of a horn relay. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of operation of a dc motor. |  |
| b) | With a neat sketch explain the construction of an over-running clutch drive. |  |
| c) | Explain the operation of a gear reduction starter. |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | CDI system |  |
| b) | Electronic spark advance |  |
| c) | Maintenance of charging system |  |
| d) | Electronic voltage regulator |  |

***********