## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(EN301)/(4137) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is universal shunt? |  |
| b) | What is the function of a delay line? |  |
| c) | What are the conditions for bridge balance? |  |
| d) | What are Signal Generators? |  |
| e) | What are Multimeters? |  |
| f) | What are standards? List them. |  |
| g) | What is meant by ‘Loading effect’? |  |
| h) | Explain few Lissajous patterns for phase measurement using CRO’s. (any 3) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of a PMMC meter. |  |
| b) | Explain series type Ohmmeter along with the necessary circuit diagrams. |  |
| c) | For a measurement system, explain its dynamic characteristics. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the sources of errors? List and explain types of errors. |  |
| b) | With the help of a neat block diagram, explain basic Cathode Ray Oscilloscope. |  |
| c) | Explain triggered sweep with respect to horizontal deflection system in a Cathode Ray Oscilloscope. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram explain Dual Trace CRO. |  |
| b) | Explain an AC bridge to find unknown Inductance and also derive the expression to calculate unknown Inductance. |  |
| c) | Explain along with a neat circuit Wheatstone’s bridge. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat block diagram working of a Signal Generator. |  |
| b) | Explain frequency selective wave analyzer with a block diagram. (Only for students of revised scheme) |  |
| c) | Explain digital frequency meter. (Only for students of revised scheme) |  |
| b) | Explain the operation of frequency synthesized signal generator using neat diagram. (Only for students of old scheme) |  |
| c) | Explain the working principle of LCD display. (Only for students of old scheme) |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Function generator |  |
| b) | Schering Bridge |  |
| c) | Cathode Ray Tube |  |
| d) | Vertical Deflection system |  |
| e) | Kelvin’s Bridge |  |

***********