## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(MN503)/(4297) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How are the underground coal mines classified based on gassiness? |  |
| b) | Differentiate between Ascensional and Descensional ventilation. |  |
| c) | Compare laminar air flow with turbulent air flow. |  |
| d) | What is Angle of Yaw? Give its significance. |  |
| e) | What do you mean by ‘operating point’ of a fan? |  |
| f) | Write short note on flexible ducts. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the physical, chemical and physiological properties of carbon monoxide. |  |
| b) | i) “Flame safety lamp is used in underground coal mines even though naked flames are prohibited”. Justify the statement. ii) Write short note on methane layering. |  |
| c) | i) What are the objectives of mine ventilation? ii) Explain the terms ‘Heat stroke’ and ‘Cooling power’ of mine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the limitations of natural ventilation pressure? Calculate the NVP of a mine with following parameters: D.C shaft – density of air 1.21 Kg/m3, 32C temp and 250m depth U.C shaft – 37C temperature 200m deep |  |
| b) | Mention the factors considered in erecting a ventilation door. Under which conditions you go for multiple doors? |  |
| c) | Derive an expression for equivalent resistance of galleries connected in parallel. What is the equivalent resistance of three galleries of resistance ‘R’ each connected in parallel? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle and construction of axial air flow fan. What are the different ways of changing the velocity of air? |  |
| b) | A radial centrifugal fan 5m in diameter is running at a speed of 100 rpm with a w.g. of 70mm. The average temperature of the downcast shaft is 20C and the barometric pressure is 750mm. Calculate the manometric efficiency of fan. |  |
| c) | The evasee chimney of a fan has an area of 4m2 at the base and 14m2 at the outlet. Calculate the saving of w.g. and of horse power due to the chimney when the output of the fan is 6000 m3/min, assume air density to be 1.2 Kg/m3. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the w.g expended and the horse power lost to over frictional resistance in ventilating a drift of size 2.5m x 3m and a length of 300m. The quantity that passes in the drift is 300m3/min. Assume coefficient of resistance K=0.00161 Kilomurg. |  |
| b) | With a neat sketch explain the overlap method of auxiliary ventilation in a long heading. |  |
| c) | Write short note on survey interval. Explain any one method of quantity survey. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | How is the percentage test conducted by Flame safety lamp to detect methane? |  |
| b) | What are the provisions of CMR 1957 with respect to main mechanical ventilator? |  |
| c) | Explain the terms i) Equivalent orifice ii) Velocity pressure. |  |
| d) | Draw a neat sketch of characteristic curves of centrifugal fan. |  |

***********