## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Advanced Microcontroller(EN613) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief how timer 0 Interrupt works. |  |
| b) | Write an instruction to enable external hardware Interrupt 0. Activate it as edge triggered. |  |
| c) | Explain in brief the importance of SBUF register in serial communication. |  |
| d) | Which register has SMOD bit? What is its significance? |  |
| e) | With crystal frequency 11.0592 MHz, find the value of TH1 for 9600 baud rate. |  |
| f) | What do you mean by embedded system? |  |
| g) | Draw interfacing diagram of 4x4 matrix keyboard with 8051 microcontroller. Assume ports for Input, Output. |  |
| h) | State different features of PIC µc 18FXX. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 8 switches are connected to P0 & 8 LEDS are connected to P1 of 8051. Write an assembly language programme to read status of switch of P0 & display it on P1 for time delay 20msec. Crystal frequency = 12 MHz. |  |
| b) | Write an assembly language programme to generate a square wave of 5 KHz on P1.3 & 25 KHz on P2.3 bit of 8051 by using timer 0 & 1 interrupts. Use mode 2 of timers. Assume crystal frequency = 22MHz. |  |
| c) | Write an assembly language program to read the data from P1 and write it to P2, also transmit it serially. Use serial Interrupt. Assume crystal frequency = 11.0592 MHz & baud rate is 9600. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the steps to transmit the data serially in 8051 system. |  |
| b) | Write assembly language program to receive the data which has been sent in serial form and send it to P0 in parallel form. Also save the data to RAM location 45H. Assume crystal frequency = 11.0592 MHz, baud rate 9600. |  |
| c) | Write an assembly language program to transmit ASCII letter B serially with double baud rate. Assume crystal frequency = 11.0592 MHz, baud rate 9600. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Rs-232 bus standard with its pin diagram. |  |
| b) | Write short notes on i) ICE ii) Target board iii) Device programmer. |  |
| c) | Explain in brief IDE, compiler, simulator & debugger. |  |
| d) | Draw interfacing diagram of 4, 7 segment displays to port 2 of 8051. Write assembly language program to display data on it for 20msec. Assume crystal frequency = 12MHz. (Assume data to be displayed) |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw interfacing diagram of stepper motor to 8051 port 1. Write assembly language program to rotate it clockwise continuously with full step sequence. |  |
| b) | Draw Interfacing diagram of 4x4 matrix Keyboard to P1 & P2 of 8051 as Output & Input respectively. Write assembly language program to detect & identify key activation and display on P0. (ASCII code of key should be displayed) |  |
| c) | Draw Interfacing diagram of ADC (0808) to 8051. Write assembly language program to start the conversion and store result to Accumulator. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw interfacing diagram of DAC (0808) to port 1 of 8051. Write assembly language program to generate triangular waveform. |  |
| b) | Write short notes on i) Registers of PIC 18FXX ii) Memory organisation of PIC 18FXX. |  |
| c) | i) Draw only Interfacing diagram of LCD to 8051. ii) Compare all serial protocols. |  |
| d) | Draw & explain functional block diagram of PIC 18FXX microcontroller. |  |

***********