## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI

Subject: 8051 Microcontroller(EN501) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain why ROM less Intel microcontroller exist. Name part number of atleast one. |  |
| b) | Draw typical block diagram of microcontroller and microprocessor. |  |
| c) | Name dual functions of each of port 3 pins of microcontroller 8051. |  |
| d) | Name the special function Register, which determines priority of 8051 microcontroller interrupts. Enlist all the interrupts as per their ranking, if all are set at same priority. |  |
| e) | Compare MOV, MOVC and MOVX instructions with appropriate examples. |  |
| f) | Write a program for 8051 microcontroller, to set PORT 0 bits 1, 5 and 7 to one and to set the rest of bits to zero. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the flags and program status word of 8051 microcontroller. |  |
| b) | Draw the block diagram of 8051 microcontroller and explain the functions of various blocks. |  |
| c) | Draw a neat diagram of port 2 pin configuration circuit. Explain its working in detail as i) Input pin ii) Output pin iii) Address pin. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain and write the format for TCON and TMOD special function registers. |  |
| b) | Explain the operation of mode 0 and mode 1 used for serial data transmission in 8051 microcontroller. |  |
| c) | Explain internal and external interrupts provide by 8051 microcontroller. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write instructions for 8051m microcontroller to perform following tasks: - Copy the byte at internal RAM address 27h to external RAM address 0027h - Store the contents of RAM location 20h at address contained in register R0. - Set timer1 register to A23Dh |  |
| b) | Explain the following instructions with examples: i) CPL A ii) ANL A, add iii) ANL c, b iv) Setb b v) SWAP A vi) RRC A |  |
| c) | State the contents of register A, carry flag and overflow flags. MOV A,#1Ch MOV R5,#0A1h ADD A,R5 ADD A,R5 ADDC A,#10h ADDC A,#10h | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program for 8051 microcontroller to store FFh in all the internal RAM locations from address 50h to 60h, using indirect addressing mode. |  |
| b) | Write a program for 8051 microcontroller to count number of even numbers stored in internal RAM location from 08h to 28h. Save the result at RAM location 30h. |  |
| c) | Write a program to find average of five 8 bit numbers stored from 40h to 44h in internal RAM of 8051 and store the result at 55h. Assume that the sum is 8 bit only. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the operation of stack and stack pointer of 8051 microcontroller. |  |
| b) | Write a short note on special function Register IE of 8051 microcontroller. |  |
| c) | State and explain 2 instructions of conditional jumps and 2 instructions of un-conditional jumps w.r.t. 8051 microcontroller. |  |
| d) | Write a program to rotate bytes in register R0 to R3 i.e. copy the data in R0 to R1, R1 to R2, R2 to R3 and R3 to R4 registers of 8051 microcontroller. |  |

***********