## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Machine Design Practice(ME601)/(4045) [Old/Rev]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define “factor of Safety”. Why is it provided in the design of machine parts? |  |
| b) | State the applications of the cotter joint. Why taper is provided on the cotter? What is its normal value? |  |
| c) | What is meant by creep? Draw a typical creep curve and explain the three stages of creep. |  |
| d) | A shaft transmits 20KN power and rotates at 500 rpm. The material of the shaft is 50C4 (yt = 460 N/mm2) and the factor of safety is 2. Determine the diameter of the shaft on the basis of its shear strength. |  |
| e) | Explain the steps involved in obtaining various dimensions of a muff coupling. |  |
| f) | What is meant by bolts of uniform strength? Draw 2 different bolts on uniform strength. |  |
| g) | Define the following terms with reference to springs: i) Spring index ii) Spring Rate iii) Solid length of spring |  |
| h) | Compare Ball bearing with Roller bearing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a Knuckle joint to connect two mild steel bars under a tensile load of 25KN. The permissible stresses are 65MPa in tension, 50MPa in shear and 83MPa in crushing. |  |
| b) | A Right angled Bell crank lever is subjected to a force of 7.5KN at the short arm end. The lengths of the short and long arms are 100mm & 500mm respectively. The lever and the pins are made of steel Fe E300 (yt = 300N/mm2) and the factor of safety is 5. The permissible bearing pressure on the pin is 10N/mm2. The lever has a rectangular cross section and the ratio of width to thickness is 4:1. The length to diameter ratio (l/d) of the fulcrum pin is 1.5:1. Calculate i) Diameter and length of the fulcrum pin ii) Shear stress on the pin iii) Dimensions of the boss of the lever and fulcrum pin iv) Dimensions of the cross section of the lever. Assume that the axis of Bending moment of the lever extends upto the axis of the arm of the fulcrum. | ……2/- |
|  | -2- |  |
| c) | A pulley made up of grey cast iron transmits 10KW power at 720 rpm. The diameter of the pulley is 500mm. The pulley has 4 arms of elliptical cross section in which the major axis is twice the minor axis. Determine the dimension of the cross section of the arm if the maximum allowable bending stress is 30MPa. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | A bracket made of steel Fe E200 (yt = 200N/mm2) and subjected to a force of 5KN at an angle of 30 to the vertical is shown in the fig.1. The factor of safety is 4. Determine the dimensions of cross section of the bracket. | (4) |
|  | OR |  |
| a) | Fig.2 shows a C-clamp which carries a load P of 25KN, and the cross section of the clamp is rectangular and the ratio of width to thickness (/t) is 2:1. The clamp is made of cast steel (ut = 400N/mm2) and the factor of safety is 4. Determine the dimensions of the cross section of the clamp. | (4) |
| b) | A transmission shaft is supported on two bearings which are 1m apart. Power is supplied to the shaft by means of flexible coupling, which is located to the left of left end bearing. Power is transmitted from the shaft by means of a belt pulley, 250mm diameter, which is located at a distance of 300mm from the left end bearing. The mass of the pulley is 20 Kg and the ratio of belt tensions on the tight and slack side is 2:1. The belt tensions act vertically downwards. The permissible tensile stress on the shaft material is 150 N/mm2 and the factor of safety is 3. Determine the shaft diameter, if it transmits 10KW power of 360 rpm from the coupling to the pulley. Refer fig.3. | (8) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel plate subjected to a force of 3KN and fixed to a vertical channel by means of four identical bolts as shown in figure. The allowable tensile and shear stresses for the bolt material are 180MPa and 90MPa. Determine the diameter of the bolt. Refer fig.4. |  |
| b) | A plate, 75mm wide and 10mm thick is joined with another steel plate by means of single transverse and double parallel fillet welds, as shown on fig.5. The joint is subjected to a maximum tensile force of 55KN. The permissible tensile and shear stresses in the weld material are 70 and 50 N/mm2 respectively. Determine the required length of each parallel fillet weld. Refer fig.5. |  |
| c) | Design the rectangular sunk key for transmitting 1200N-m torque to the pulley mounted on it. The permissible stresses for the shaft as well as for key are shear stress of 50 MPa and crushing stress of 95 MPa. Assume an overload factor of 1.25. | ……3/- |
|  | -3- |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Design a cast iron flange coupling to connect two shafts in order to transmit 7.5KW at 720 rpm. The permissible shear stress for shaft, bolt and key material is 33 MPa. Permissible crushing stress for bolt and key material is 60 MPa. Permissible shear stress for the cast iron is 15 MPa. Maximum torque is 20% greater than the full load torque. |  |
| b) | A screw jack has to lift a load of 60KN through a height of 300mm. The permissible stresses for steel screw material are t = c = 180 MPa &  = 90 MPa. The nut is made of Phosphor Bronze material with permissible stresses t = 100 MPa, c = 90 MPa and  = 80 MPa. The permissible bearing pressure between nut & screw is 20 MPa. Determine i) Diameter of screw ii) Dimensions of the nut. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a compression helical spring to carry a load of 800N. The deflection is 30mm. Take spring index C=10 and Wahl’s correction factor = 4C−14C−4+0.615C Assume permissible shear stress as 200 MPa and G = 84 KN/mm2. |  |
| b) | A semi-elliptical spring has 6 leaves each 40mm wide to carry a central load of 5KN. Determine the thickness of each leaf, if permissible bending stress (b) is 250 MPa. Also, find the deflection. Take E = 2.1 x 105 N/mm2. |  |
| c) | Write short notes on: - Applications of Sliding and Rolling contact bearings - Various thread forms used in Power Shears |  |
|  |  | ……4/- |