## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Diesel Engines Operation & Maintenance(SH722)/(5045) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a multigrade oil? Why is the need for using it in diesel engine? |  |
| b) | Make a list of the important startup checks of a diesel engine on a barge. |  |
| c) | What is the need for engine overhauling? How does it restore the engine performance? |  |
| d) | Draw a neat sketch of a Piston ring expander and state its use in engine service. |  |
| e) | What is engine valve lapping? How is the process carried out? |  |
| f) | What is troubling shooting? What is the use of a troubleshooting chart? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between naturally aspirated and a turbocharged engine. Explain with a simple sketch the working of a cooling system on a diesel engine. |  |
| b) | With a neat flow diagram, explain the pressurized lubricating system of a diesel engine used to power a crane. |  |
| c) | What precautions need to be taken prior to stopping a diesel engine? List the various engine operating parameters and state the importance of keeping the records. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain crankcase explosion of large diesel engines. What precautions are taken to avoid the same? |  |
| b) | Explain with a neat sketch the operation of a Jerk –type fuel injection pump. |  |
| c) | What is the need for setting valve-tappet clearance? How is valve tappet clearance adjusted? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | ‘Engine emits black smoke’. Prepare a troubleshooting chart for the above mentioned problem. |  |
| b) | Prepare a list of preventive maintenance checks of the cooling system of the diesel engine. |  |
| c) | How is the operational and maintenance manual useful for diesel engine operation and maintenance? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the step-by-step procedure for any one assembly - Cylinder head and Engine block - Piston and connecting rod |  |
| b) | For what possible defects is the cylinder liner inspected OR Crankshaft inspected? |  |
| c) | What is major & minor overhauling? Enumerate on the safe working practices during overhauling a diesel engine. | …..2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Piston cooling |  |
| b) | Bearing clearance measurement |  |
| c) | Torque wrench & Filler gauges |  |
| d) | Applications of Diesel Engines |  |

***********