## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-II(2006) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the assumptions made in analysis of structures? |  |
| b) | Explain conservation of momentum with an example. |  |
| c) | What are the advantages of banking of curves and superelevation? |  |
| d) | What is the relationship between linear and angular displacements, velocity and acceleration? |  |
| e) | Define kinetic energy and potential energy. What are their units? |  |
| f) | Find the maximum r.p.m. with which a stone tied to a string, of 1.5 Kg mass can be rotated in a horizontal circle of radius 1m. Breaking strength of the string attached to the stone is 600N. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A projectile shot at an angle of 60 above the horizontal, strikes a building 25m away at point 15m above the point of projection. Find the velocity of projection. g = 9.8m/sec2 |  |
| b) | A wheel 80cm in diameter, starts from rest and accelerates uniformly. After 24 seconds it attains angular velocity of 120 rad/sec. Find the angular acceleration and the total angle turned through. |  |
| c) | A Flywheel starting from rest, accelerates uniformly. In the 6th second of its motion it turns through 3/4th of a revolution. Find the angular acceleration and angular velocity after five seconds. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An electric train has 800 tonnes mass. The resistance to motion is 1% of train mass. The electric motors can provide a tractive force of 200KN. What is the acceleration and time take to reach velocity of 90Km per hour. |  |
| b) | Two bodies of mass 80 Kg and 20 Kg are connected by a string and move along rough horizontal plane under the action of applied horizontal pull of 400N to 80 Kg mass. The coefficient of friction between blocks and plane is 0.3. Find the acceleration and tension in the string. |  |
| c) | An elevator of mass 2000 Kg is to be lifted and lowered by means of a rope. Find the tension in the rope when the lift is i) moving upwards with an acceleration of 2m/sec2 ii) moving downwards with an acceleration of 1.5m/sec2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A trolley of mass 200 Kg moves on a level track for a distance of 500m. If the resistance of the track is 100N, find the work done in moving the trolley. Find the work done if the trolley moves on an inclined track of 1 in 250. |  |
| b) | A well 4m in diameter and 12m deep is half full of water. It has to be emptied in 10 hours by means of a pump to an average height of 30m. If the efficiency the pump in 60% determine the rated power of the pump. | ……2/- |
|  | -2- |  |
| c) | A body has 220 KJ of kinetic energy at a velocity of 750 m/s. How much kinetic energy will be lost if velocity comes down to 500 m/s? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of Double Purchase Crab and write an expression for its velocity ratio. |  |
| b) | In a lifting machine, whose velocity ratio is 50, an effort of 100N is required to lift a load of 4KN. Is the machine reversible? If so what effort should be applied, so that the machine is at the point of reversing? |  |
| c) | In a double threaded Worm and Worm Wheel, the number of teeth on the worm wheel is 50. The diameter of the effort wheel is 200mm and that of the load drum is 100mm. Calculate the velocity ratio. If the efficiency of the machine is 20%, determine the effort required to lift load of 30N. |  |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | Using the method of joints or method of sections, find the forces in the member at joint D of the trusses supported at A & B. Refer fig.(i) |  |
| b) | Using method of joints or methods of sections find the forces in all the members in the cantilever truss. Refer fig.(ii). |  |