## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any 5 from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Weight distribution curve for ship shaped vessel |  |
| b) | Deck stringer and shear strake |  |
| c) | Buoyancy curve |  |
| d) | Hogging and sagging bending moment |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain the procedure of drawing shell expansion diagram. | (6) |
| b) | Detail the importance of shell expansion in shipyard while constructing a ship. | (6) |
| Q.No.3. | Sketch the midship section of a bulk carrier showing all structural members. | (12) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain briefly by taking example of a box shaped vessel how the section modulus of the ship can be increased by adding minimum area to the cross-section. | (8) |
| b) | Explain to what extent the section modulus of a ship is increased and what is safety factor. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Explain with neat sketch difference between bending moment curve for a ship poised on a wave profile with crest at mid-ship and trough at ends and vice versa. | (8) |
| b) | Explain how a wave profile is obtained for bending moment calculations. | (4) |
| Q.No.6. | A box shaped vessel of uniform construction is 30m long and displaces 352 tonnes when empty. It is divided by transverse bulkheads into four equal compartments. Cargo is loaded into each compartment and level stowed as follows: No.1 hold -192 tonnes No.2 hold -224 tonnes No.3 hold -272 tonnes No.4 hold -176 tonnes Draw the shear force and bending moment diagram. | (12) |
| Q.No.7. | Calculate the following for a midship section shown in Fig.1: - The position of neutral axis - Moment of Inertia of the total area of the material about the neutral axis - Section modulus at the deck and keel | (12) ……2/- |