## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Engineering & Technology

Subject: Engineering Mechanics(CS301) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the effects of forces? |  |
| b) | What are the characteristics of a couple? |  |
| c) | State and explain Lami’s theorem with example. |  |
| d) | Define co-efficient of friction, angle of friction and cone of friction. |  |
| e) | Define and explain work and power. |  |
| f) | Explain Varignon’s theorem. |  |
| g) | What is Perfect truss and Imperfect truss? Explain with examples. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the resultant in magnitude and direction for following concurrent force system: i) 250N acting 30 N of East ii) 350N acting due South iii) 450N acting 60 S of West iv) 550N acting due West. |  |
| b) | Find the resultant in magnitude, direction and find its position from A. Refer fig.1. |  |
| c) | Find the resultant of parallel force system and its position from A. Refer fig.2. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) a) | Using Lami’s theorem, find the forces in the members AB, BC, CD and find the unknown W. Refer fig.3. OR Find the reaction at A and B of a simply supported beam. Refer fig.4. |  |
| b) | Find the forces in the members of cantilever truss. Refer fig.5. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An object of weight 100N is stopped from moving down on a plane inclined at 30 to the horizontal by a horizontally applied force ‘P’. The coefficient of friction of the surface of the plane is 0.25. Determine the minimum magnitude of force ‘P’. |  |
| b) | Determine the angle  at which the ladder will start slipping if  at floor and wall is 0.2. Assume weight ‘W’ of the ladder acts at centre of the ladder of span ‘l’. |  |
| c) | A vehicle of mass 2000 Kg has a velocity of 45 Kmph. On its way it hits a car parked by the road side. In doing so its velocity is reduced to 36 Kmph. If the mass of the car is 250 Kg, find its velocity after the impact. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two weights 90N and 105N are connected by a thin weightless string passing over a light frictionless pulley. Find tension in the string and acceleration of the system. | …..2/- |
|  | -2- |  |
| b) | A wire rope supports a lift of mass 2000 Kg. What is the tension in the rope if the lift is i) moving up with an acceleration of 0.8 m/sec2 ii) moving down with a acceleration of 1.2 m/sec2. |  |
| c) | A train of 3000KN weight starts from rest and reaches a velocity of 54 Kmph after one minute. If the frictional resistance of the track is 5N per KN of the train weight, find the tractive force developed by the engine. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An engine pulls a train with uniform velocity 36Kmph and exerts a tractive pull of 12KN. Find the work done by the engine in 10 minutes. |  |
| b) | A pump reduces water level in well 3m in diameter by 1m in 20 minutes and lifts the water through average height of 5m. If the efficiency of the pump is 80%, find the rated power of the pump. |  |
| c) | i) A body is lifted at a point 5m above the ground level. What is its Potential energy if the mass of the body is 20 Kg? ii) Define Kinetic Energy and state conservation of energy with an example. |  |