## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg./C/CT

Subject: Design of Steel Structures(FE701)/ Design of Structures-I(Steel)[4024]

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-1984 and steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Compare the properties of mild steel and high tensile steel. |  |
| b) | State the main advantages of steel as a structural material. |  |
| c) | What are the sections that are normally used as purlins or girts? How is the spacing of purlins fixed? |  |
| d) | What is the difference between bending and buckling of a beam member? |  |
| e) | What is the main purpose of lacings and battens? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a lap joint between two plates of size 100x16mm thick and 100x10mm thick so as to transmit a load of 75 KN using a single row of turned and fitted bolts. |  |
| b) | Two plates 160mmx10mm and 160mmx12mm are to be butt welded. Calculate the strength of the welded joint in tension if i) a single V butt weld ii) a double V butt weld is used to connect them. Take permissible stress in weld as 150N/mm2. |  |
| c) | Design a lap joint to connect two plates 130x10mm and 130x12mm by fillet welds. Take tensile stress 150N/mm2 and shear stress in weld 110 MPa. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Tie member in a truss is 2.50m long and is required to carry a pull of 150KN. Design using a single unequal angle with longer leg connected. Take shear stress = 108Mpa for welds. | (7) |
|  | ### OR |  |
| a) | Compare the tensile strength capacities of 2 ISA 100x100x8mm when connected on i) Same side of gusset plate ii) Either side of gusset plate. Assume gusset plate of 10mm thickness and members are tack welded in both the above cases. | (7) |
| b) | Determine the tensile strength of a roof truss tie member ISA 150x75x12mm connected to a gusset plate by 8mm fillet weld. | (5) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Determine the axial load carrying capacity of a single channel ISMC 100 with its effective length equal to 1.70m. | (5) |
|  | ### OR |  |
| a) | Briefly explain the design procedure for slab base. | (5) |
| b) | Design a column to support an axial load of 1400KN using two channel sections placed with its web back to back, the effective height of the column is 5.0m. | (7) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam is to carry a load of 2.5KN/m. The beam ends are resting on 250mm thick walls with bearing plates with clear span of 6.0m. The beam supports a floor slab. Design the beam. Take fy = 250MPa. |  |
| b) | Overhang beam with 5m main span and 1.50m overhang cantilever span is subjected to a udl of 28KN/m. Design a standard rolled beam. The top flange of beam is laterally supported. |  |
| c) | A beam consisting of ISMB 300 @ 44.2 Kg/m is simply supported over a span of 6.0m. Determine the safe load the beam can carry. Assume that the beam is laterally supported. Take fy = 250N/mm2 and E=2x105 N/mm2. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Design an I section purlin for the following data: - Spacing of roof trusses 4.0m - Spacing of purlins 1.90m - Pitch of roof 1/4.5 - Weight of GI sheeting 133N/m2 - Wind load intensity normal to roof 1500N/m2 | (8) |
| b) | Draw line diagrams of various types of roof trusses stating their use. | (4) |
|  | ### OR |  |
| b) | State the advantages and disadvantages of using tubular and box sections in trusses. | (4) |

***********