## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/MED/EI

Subject: Basic Electronics Engineering(CS308) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw output waveform of a full wave rectifier & explain the concept of ripple voltage. |  |
| b) | Explain reverse breakdown characteristics of Zener diode. |  |
| c) | In common base configuration, the emitter current is 1mA. If emitter circuit is open, the collector current is 50µA. Find total collector current if  is 0.92. |  |
| d) | Explain the construction of PNP transistor. |  |
| e) | Define the following terms of JFET: i) Transconductance ii) Amplification factor. |  |
| f) | Differentiate between ROM & EPROM. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram & waveforms explain the operation of capacitor filter. |  |
| b) | Explain the operation of Zener diode regulator circuit. |  |
| c) | A Zener diode regulator has Vz = 10V, Izm = 32mA, series resistance of 1KΩ & input voltage is 50V. Determine the range of RL that will ensure that output voltage is 10V. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the basic operation of NPN transistor. |  |
| b) | Draw & explain the input characteristics of common emitter configuration. |  |
| c) | Define  & β of the transistor & derive the relationship between them. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & operation of P-channel JFET. |  |
| b) | Explain Drain & transfer characteristics of N-channel depletion type MOSFET. |  |
| c) | Draw & explain the construction of N-channel enhancement type MOSFET. Also draw symbol of the same. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of TTL NOR gate with neat diagram. |  |
| b) | Explain following characteristics of digital logic IC: i) Propagation delay time ii) Power dissipation iii) Noise immunity |  |
| c) | State the features & applications of RAM, ROM & EEPROM. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Specifications of Zener diode |  |
| b) | Leakage currents in transistor |  |
| c) | Difference between BJT & FET |  |
| d) | Static & Dynamic memories |  |

***********