## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(FE601)/(4022) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are Principal planes and Principal stresses? |  |
| b) | Explain the limit of eccentricity for rectangular section, so as not to develop tension anywhere. |  |
| c) | Explain with example what are statically indeterminate structures. |  |
| d) | How are Influence Line Diagrams drawn for simply supported beam for reaction at supports? |  |
| e) | What is the core of a solid circular section of diameter D subjected to eccentric load? |  |
| f) | Determine the degree of indeterminacy for two hinged arch and propped cantilever. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A short hollow pier of 1.2m square section outside and 1.0m square section inside is subjected to a direct load of 120KN along its outer edge point at a point along axis. Determine the final stresses at the base of the pier. Draw neat sketch of stress distribution diagram. |  |
| b) | A short masonry pillar is 600mm x 600mm in section. The pillar carries a point load of 1000KN acting on the centroidal axis of the section and at an eccentricity of 80 mm from the longitudinal axis. Find the maximum and minimum stresses on the section. |  |
| c) | A column is 400mm x 300mm in cross-section. It carries a load of 120KN acting on the diagonal of the section at a distance of 50mm from the centre. Find the stresses at the four corners of the section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The principal tensile stresses at a point across two perpendicular planes are 80N/mm2 and 40N/mm2. Find the normal and tangential stresses and the resultant stress and its obliquity on a plane at 20º with the major principal plane. |  |
| b) | A point in a strained material is subjected to a tensile stress of 60N/mm2 and a compressive stress of 40N/mm2, acting on two mutually perpendicular planes and a shear stress of 10N/mm2 are acting on these planes. Find the normal stress, tangential stress and resultant on a plane inclined to 30º with the plane of the compressive stress. |  |
| c) | A bar is subjected to a tensile stress of 80N/mm2 along with shear stress of 30N/mm2. Determine the principal stresses and the principal planes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A uniformly distributed live load of 6KN/m run of length 5m moves on a girder of span 16m. Find the maximum positive and negative shear force at a section 6 m from the left end. Use Influence Line Diagram. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | Two wheel loads 8KN and 20KN spaced 2m apart move along a girder of span 16m. Find the maximum positive and negative shear force at a section 4m from the left end. Any wheel load can lead the other. Use Influence Line Diagram. |  |
| c) | Draw ILD for simply supported beam of span L for i) Shear force at any section at ‘a’ from left support ii) Bending moment at any section at ‘a’ from left support. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain the concept of pure torsion with an example. |  |
|  | ii) What are the assumptions made in the theory of pure torsion? |  |
| b) | A steel shaft transmits 105KW at 160 rpm. If the shaft is 100mm in diameter, find the torque on the shaft and the maximum shearing stress induced. Find also the twist of the shaft in a length of 6m. Take modulus of rigidity as 8 x 104 N/mm2. |  |
| c) | A hollow shaft of external diameter 50mm and wall thickness 5mm transmits a torque of 10 KN-m. Find the maximum shear stress induced in the shaft. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Differentiate between statically determinate and statically indeterminate structure with examples. | (6) |
|  | Only for students of Old scheme: |  |
| b) | A flitched beam consists of wooden joist 150mm wide and 300mm deep strengthened by steel plate 12mm thick and 300mm deep one on either side of the joist. If the maximum stress in the wooden joist is 7 N/mm2, find the corresponding maximum stress attained in steel. Find the moment of resistance of the section. Take ES = 20 EW | (6) |
|  | OR |  |
| b) | A flitched beam consists of a wooden joist 150mm wide and 250mm deep strengthened by steel plates 10mm thick and 200mm deep one on either side of the joist. If the stresses in wood and steel are not to exceed 7N/mm2 and 120N/mm2, find the moment of resistance of the section of the beam. Take ES = 20EW | (6) |
|  | Only for students of Revised scheme: |  |
| b) | A thin cylindrical shell of thickness 5mm and diameter 350mm is subjected to an internal pressure which produces a strain of 1/2500 in the diameter. Find this internal pressure and the consequent hoop and longitudinal stress. Take E= 2x105 N/mm2 and 1/m = 0.3 | (6) |
|  | OR |  |
| b) | A pipe of 200mm internal diameter and 100mm thickness contains a fluid at a pressure of 6N/mm2. Find the maximum and the minimum hoop stress across the section. | (6) |

***********