## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are requirements of good lighting? |  |
| b) | Define i) Plane angle ii) Solid angle iii) Luman. |  |
| c) | Define i) D.L.O.R. ii) U.L.O.R. |  |
| d) | What is meant by space to mounting height ratio? |  |
| e) | State any three important features of sports lighting? |  |
| f) | Explain how streetlight is different from indoor lighting? |  |
| g) | What is Flood lighting? Where it is used? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain what is indirect and semi-indirect lighting system. |  |
| b) | What is glare? How glare is produced? Also suggest how it can be avoided? |  |
| c) | With the help of neat diagram, explain the difference between specular reflection and diffuse reflection. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by Polar curve? Also mention its advantages. |  |
| b) | State and explain the two laws of illumination. |  |
| c) | Two lamp posts are 16 m apart and are fitted with a 100 CP lamp each at a height of 6 m above ground. Calculate the illumination on the ground i) under each lamp ii) midway between the lamps. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out all the steps used for indoor lighting design. |  |
| b) | Explain what is i) coefficient of utilisation ii) Maintenance factor. Discuss the factors on which they depend. |  |
| c) | Estimate the number and wattage of lamps which would be required to illuminate a workshop space 60 x 15 metres by means of lamps mounted 5 m above working plane. The average illumination required is about 100 lux, coefficient of utilization =0.4, luminous efficiency 16 lumens per watt. Assume a space–height ratio of unity and a candle power depreciation of 20%. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the two general principles employed in the design of street lighting installations. |  |
| b) | Explain what is i) Cut-off luminaire ii) Semi-Cut off luminaire iii) Non cut-off luminaire. Also list their applications. |  |
| c) | State and explain various arrangements for street lighting with their applications. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain in details the steps involved in Flood lighting calculations. |  |
| b) | Write a short note on special features of Auditorium lighting. |  |
| c) | A building frontage 50 m x 15 m is to be illuminated by flood lighting projectors situated 25 m away. If the illumination is 100 lux, coefficient of utilization 0.5, depreciation factor 1.5, waste light factor 1.2, estimate the number and size of projectors. Sketch the projectors recommended indicating the usual adjustment provided. |  |

***********