## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical & Electronics Engg.

Subject: Power Generation, Transmission & Distribution(5091)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss why pulverised coal is used in steam power plants. |  |
| b) | State the significance of higher values of diversity factor and demand factor. |  |
| c) | What do you mean by sag in overhead lines? State the factors affecting sag. |  |
| d) | What do you mean by string efficiency? Why should string efficiency have a high value? |  |
| e) | State any three IE rules relevant to domestic wiring. |  |
| f) | Explain the concept of distributor and service mains. |  |
| g) | State any three IE rules for earthing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the factors on which choice of number and rating of units for a given load curve depends. |  |
| b) | Explain in brief the main components and their functions of a stand by diesel generator set. |  |
| c) | A Station has following daily load cycle: Time (hrs) 6-8 8-12 12-16 16-20 20-24 24-6 Load (MW) 20 40 60 20 50 20 Plot the load curve and calculate (i) no of units generated daily (ii) load factor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the different causes of failure of insulators of transmission lines. |  |
| b) | What do you mean by corona? Give methods for reduction of corona loss. |  |
| c) | State the advantages and disadvantages of HVDC transmission system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the following: i) radial system ii) ring main system iii) interconnected system. |  |
| b) | Explain the classification of distribution system with respect to voltage and number of wires. Briefly explain AC three phase, four wire system of distribution. |  |
| c) | Write a short note on system voltage and permissible voltage drop in various parts of urban and rural distribution system. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram explain plate earthing. |  |
| b) | Briefly explain PVC casing capping wiring and state its advantages. |  |
| c) | A classroom is to be wired using PVC casing capping and consists of following points: (i) light points = 5, 60 watts each (ii) fan points = 2, 60 watts each. (iii) plug points = 1, 1000 watts. Draw the wiring plan for above and estimate the total material required. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are bundled conductors? Slate their advantages. |  |
| b) | Write a short note on the following: (i) Steel towers (ii) Methods to reduce earth resistance. |  |
| c) | With a neat diagram explain: (i) Stair case wiring (ii) Godown wiring. |  |

***********