## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg.

Subject: Electrical Engineering Materials(4101)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why copper is used for the windings of electrical machines even through it is expensive? |  |
| b) | List out the special properties of Nichrome and hence give its area of applications. |  |
| c) | Explain an intrinsic semiconductor. Name a few intrinsic semiconductors. |  |
| d) | Explain the term dielectric strength of an insulating material. Mention its unit. State the factors which affect it. |  |
| e) | Explain the terms: - Volume resistance and - Surface resistance related to insulating materials. |  |
| f) | What information does the magnetisation curve and hysteresis curve provide. |  |
| g) | Write a short note on ‘Contract materials’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a)i) | Explain the meaning of the term super conductors and give their applications. |  |
| ii) | List down the special properties of Brass and state its area of applications. |  |
| b) | State advantages & disadvantages of Aluminum as a conductor material over copper. |  |
| c) | Explain in detail various properties of conductor materials. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of energy diagram n-type and p-type semiconductors. |  |
| b) | Explain briefly the Hall effect and Hall effect generator. State the merits of semiconductor devices used in electrical industry. |  |
| c)i) | Write a short note on ‘Piezo electric materials’ and their applications. |  |
| ii) | Give the commonly used materials for i) Varistors ii)LDR iii) LCD. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the conduction through solid dielectrics. |  |
| b) | Discuss in details as regards to construction, salient feature, precaution while connecting & applications of Electrolytic capacitor. |  |
| c)i) | Explain the process of polarisation in dielectrics. |  |
| ii) | Explain with the help of a phasor diagram dielectric loss. List down the factors affecting it. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the thermal and mechanical properties of insulating materials. |  |
| b)i) | State the properties of Air and Hydrogen as coolant. |  |
| ii) | Discuss the properties of PVC as an insulating material. List some of the applications. |  |
| c) | What is the necessity for impregnating paper? List down the important physical properties of paper. Mention the various types of paper along with their characteristics. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following: i) Permeability ii) Magnetostriction iii)Curie point and iv)Susceptibility. |  |
| b)i) | List down the properties and applications of soft and hard magnetic materials. |  |
| ii) | Define the term temperature coefficient with examples. |  |
| c) | State the properties and applications of fluorescent and phosphorescent materials. |  |

***********