## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables and Mollier’s chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the functions of fusible plug. |  |
| b) | Explain the term “Entropy of steam”. |  |
| c) | Define Brake thermal efficiency and write its mathematical expression. |  |
| d) | Define thermal conductivity of a material. |  |
| e) | What is meant by ‘Vaccum efficiency’ of a condensing plant? |  |
| f) | Define one tonne of refrigeration. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch, construction of Bab-cock and Wilcox boiler. |  |
| b) | Calculate the internal energy of 1kg of steam at a pressure of 10 bar when the steam is 0.9 dry. The volume of water may be neglected. |  |
| c) | What are the advantages of artificial draught system over natural draught system? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the various methods of governing of steam turbine. Explain any one with a neat sketch. |  |
| b) | Explain with the help of suitable sketch, the working of a four stroke diesel engine. |  |
| c) | Sketch the theoretical indicator diagram for a single stage, single cylinder reciprocating compressor with clearance volume showing the various processes. Explain the need of clearance volume, in reciprocating compressor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A metal pipe having an external diameter of 150mm carries steam at 200C. The pipe is covered by a layer of 25mm thick insulating material whose conductivity is 0.21 W/mk. If the outer surface is at 100C, find the amount of heat lost per metre length per minute. |  |
| b) | The inlet and outlet temperatures of cooling water in a condenser are 27C and 35C respectively. If the vacuum in the condenser is 700 mm of Hg against barometric pressure of 760mm of Hg, calculate the efficiency of the condenser. |  |
| c) | Explain vapour compression refrigeration system with a neat sketch of T-S diagram. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are advantages of using an economizer? Explain the construction of an economizer with a neat sketch. |  |
| b) | Using Mollier chart, find enthalpy drop and final condition of steam when it is expanded isentropically from an initial pressure of 40 bar and 380C to a pressure of 1 bar. |  |
| c) | An engine uses 6.5 Kg of oil per hour of calorific value 30000 KJ/Kg. if the Brake power of the engine is 22 KW and Mechanical efficiency is 85%, calculate indicated thermal efficiency. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Multi stage compressor |  |
| b) | Heat transfer by convection |  |
| c) | Jet condenser |  |
| d) | Ideal refrigerant |  |
| e) | Latent heat load |  |
| f) | Losses occurring in a steam turbine |  |

***********