## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the function of nozzle used with steam turbines. |  |
| b) | Enumerate the different losses in a steam turbine. |  |
| c) | What do you mean by vacuum efficiency of a condenser? |  |
| d) | What is meant by Turbo charging? Give its advantages. |  |
| e) | Define the following terms: i)Co-efficient of performance ii) Degree of saturation iii) Wet bulb temperature. |  |
| f) | What is gas turbine? How is it different from steam turbine? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Dry saturated steam at 10 bar is expanded isentropically in a nozzle to 0.1 bar. Using steam tables only, find the dryness fraction of the steam at exit. Also find the velocity of steam leaving the nozzle when i) Initial velocity is negligible and ii) Initial velocity of the steam is 135 m/s. |  |
| b) | The following data relates to a single stage impulse turbine. Steam velocity = 600 m/s; Blade speed = 250 m/s; Nozzle angle = 20; Blade outlet angle = 25. Neglecting the effect of friction, calculate the absolute velocity of steam leaving the blade and the work developed by the turbine for the steam flow rate of 20 Kg/s. Also calculate the axial thrust on the bearings. |  |
| c) | Describe with a neat sketch, working of a surface condenser. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the methods of Governing a steam turbine? Describe any one method of governing steam turbines. |  |
| b) | Describe Regenerative feed heating. List its advantages. |  |
| c) | Describe with a neat sketch a low level jet condenser of the counter flow type. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch forced draught type of cooling tower. |  |
| b) | With a neat sketch explain Battery ignition system in I.C. engines. |  |
| c) | Compare two stroke & Four stroke I.C. engines. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain with a neat sketch water cooling system used in I.C. engines. |  |
| b) | Explain with a neat sketch, working principle of 4 stroke petrol engine. |  |
| c) | Following observations were taken during the trial of a single cylinder, four stroke, oil engine, running at full load. Area of indicator diagram = 300 mm2, length of the diagram = 40 mm; spring stiffness = 1 bar/mm; speed of the engine = 400 rpm; Brake load = 400 N; Spring balance reading 50N; Diameter of brake drum = 1.2m; Fuel consumption per hour = 3 kg; Calorific value of fuel = 42,000 KJ/Kg; cylinder diameter = 160mm; Stroke = 200 mm. Find the indicated power, brake power, mechanical efficiency and brake thermal efficiency. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the closed cycle gas turbine plant. |  |
| b) | Explain with a neat sketch vapour absorption refrigeration system. |  |
| c) | With a neat sketch explain the working of Ice plant. |  |

***********