## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel)[4024]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-1984 code, Steel Tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Explain briefly different types of loads considered in the design of steel structures. |  |
| b) | With neat sketches explain various types of welded joints used in steel structures. |  |
| c) | With the aid of neat sketch differentiate between Slab base and Gussetted base. |  |
| d) | Explain how do you consider effect of wind load on roof trusses. |  |
| e) | Briefly explain importance of lacings and battens for compression members. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Design a welded joint for connecting two plates of 125mm x 10mm and 100mm x 8mm by fillet welds. Take shear stress in weld 110 MPa and tensile stress as 150 MPa. | (4) |
| b) | Design a fillet weld for connecting single angle section ISA 90x60x8 mm with longer leg connected to gusset plate of 10mm. Take shear stress 110 MPa and tensile stress of member 150 N/mm2. ### OR | (8) |
| b) | Describe the procedure involved in the design of eccentric loaded welded joint to connect bracket plate to a column subjected to moment in same plane of welds. | (8) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a tie member using two unequal angle sections to carry a pull of 200 KN. Angles placed on either side of gusset plate connected by fillet welds without tack welds. |  |
| b) | A Tee section used as Tie member is ISJT 100 @ 49.3 Kg/m connected to gusset plate by fillet welds. Find tensile strength of member. |  |
| c)i) | Draw neat sketches of various types of steel sections used as tension members. | (3) |
| ii) | State the necessity of slenderness ratio check in tension members and code provisions for same | (3) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a single equal angle strut, 2.2m long, to carry a compressive load of 125KN with welded end connection. Apply usual check. Also design welds. Take shear stress of weld =100 MPa. |  |
| b) | Design a Slab base for a column ISHB 350 @ 710 N/m to carry a compressive load of 700 KN. Take bearing pressure of concrete as 4000 KN/m2 and SBC of soil as 250 KN/m2. |  |
| c) | Explain the design procedure of lacings in compression members. | ….2/- |

-2-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | A floor consists of steel beams 4m apart simply supported over effective span of 6.3 meters. The top chord of beams are encased in the floor slab of 140 mm thick. The floor has to support a superimposed load of 4 KN/m2. Design the beam and carry out checks for shear, deflection and web crippling. ### OR | (8) |
| a) | Design a main floor beam supporting three equidistant floor beams over a span of 12m. Each floor beam transmits 40KN on the main beam. Check the beam section for shear and deflection. | (8) |
| b) | Explain Web Buckling and Web Buckling in beams. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | Design a channel section purlin for a roof truss with following data : - Spacing of trusses =4.5m c/c - Spacing of purlins along the slope of truss = 1.8m c/c - Slope of truss = 28. - Wind load normal to roof surface = 1200 N/m2 v)Weight of roof sheets = 180 N/m2. OR | (8) |
| a) | Select suitable roof truss for effective span of 12m for an industrial shed with rise of truss as one-fourth of span. Determine dead loads and live loads at each panel point of truss. Assume weight of roof coverings 160 N/m2. | (8) |
| b) | Briefly explain design procedure of base plate at support of roof truss. | (4) |

***********