## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat labelled sketch showing layout of the transmission system of a 2WD automobile. |  |
| b) | Distinguish between pitching and rolling of an automobile. |  |
| c) | What is wheel alignment? Why and how is it done? |  |
| d) | Explain in brief the functions of IAT and MAP sensors in an MPFI engine. |  |
| e) | State the advantages of constant mesh gearbox over sliding mesh gearbox. |  |
| f) | Where is the condenser of a car air-conditioning system placed? How is it cooled? |  |
| g) | List the various types of wrenches and state their uses. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a monocoque construction chassis and state its advantages over the framed chassis. |  |
| b) | With the help of a neat sketch, explain the construction and working of a hydraulic shock absorber. |  |
| c) | With the help of a neat sketch, explain the working of integral type hydraulic power steering. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat-labelled sketch, explain the fuel system of an MPFI engine with two groups of injection valves. |  |
| b) | Draw a neat diagram showing injection timing chart for a 6-cylinder MPFI engine with two groups of injection valves. Explain the chart. |  |
| c) | State the location and explain the function of each of the following sensors in an MPFI engine: - Engine speed sensor - Oxygen sensor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of a synchromesh device and explain its working in a synchromesh gearbox. What is the advantage of synchromesh gearbox over constant mesh gearbox? |  |
| b) | Explain the principle of automatic transmission with the help of a schematic diagram of layout of Borg Warmer automatic transmission. |  |
| c) | Draw a neat sketch of a transfer case, showing power flow with 2WD and high gear engaged. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a neat circuit diagram, explain how a stroboscope is used to adjust the ignition timing of a petrol engine. |  |
| b) | Draw the wiring circuit diagram for the lighting system of an automobile. Label the various lights and switches. Where is the stop light switch fitted? |  |
| c) | Explain the necessity of a starting drive in an automobile. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a thermostatic cycling switch in the control of air conditioning system of a car. |  |
| b) | What are the provisions under section 56 of the Motor Vehicle Act 1988? |  |
| c) | With the help of a neat sketch, explain the use of a suitable gauge to inspect the wear on an engine cylinder bore. Name the gauge. |  |

***********