## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from

the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Dynamic Stability |  |
| b) | Damaged Stability |  |
| c) | Lost Bouyancy method |  |
| d) | Bale and Grain capacity |  |
| e) | GRT and NRT |  |
| f) | Weight curve |  |
| Q.No.2. | Answer the following: |  |
| a) | An inclining experiment was carried out on a ship of 8000 tonne displacement. A mass of 10 tonne was moved 14 m across the deck causing a pendulum 8.5 m long to deflect 110mm. The transverse metacentre was 7.15m above the keel. Calculate the height of the centre of gravity above the keel. | (05) |
| b) | The KN values of a ship are as follows: Angles; 0; 15; 30; 45; 60; 75; 90. KN; 0; 2.5; 4.94; 6.88; 8.20; 8.89; 8.94. Plot the GZ curve for KG = 9m and determine the vanishing angle of stability. | (07) |
| Q.No.3. | Answer the following: |  |
| a) | Explain Tipping and stern lifting? What are the remedies to avoid structural damage on the ship and shore caused by tipping and stern lifting during launching. | (06) |
| b) | Explain the advantages and disadvantages of side launching. | (06) |
| Q.No.4. | Answer the following: |  |
| a) | Explain the precautions taken during the conduct of inclining experiment. | (06) |
| b) | A box shaped vessel 75m x 12m is floating upright in salt water on an even keel at 2.5 m draft F and A. The forepeak tank is 7m long and has permeability 95%. Find the final draft if the forward compartment is bilged. | (06) |

………2/-

-2-

| Q.No.5. | Answer the following: |  |
| --- | --- | --- |
| a) | What is a tonnage certificate? What is the purpose of carrying a tonnage certificate? Who issues the tonnage certificate? | (06) |
| b) | Calculate GRT and NRT of a luxury Launch with the following data: Length = 50m Breadth = 10 mtrs. Depth = 3.8 m Loaded draft = 1.8 m Total enclosed volume = 1700 m3 No. of passengers = 250 Crew members = 8. | (06) |
| Q.No.6. | A Box shaped barge of length 50m is of uniform construction and has displacement of 600 tonnes when empty. The barge has equal hold lengths with loading as shown below: Hold No.5; Hold No. 4; Hold No.3; Hold No. 2; Hold No. 1. 400 tonnes; 300 tonnes; 400 tonnes; 500 tonnes; 300 tonnes. Draw i) Shear force diagram ii) Determine the position of BMmax. | (12) |
| Q.No.7 | Answer the following: |  |
| a) | Explain sagging and hogging conditions of ship in waves. | (04) |
| b) | Explain the use of bonjean curves in flooding, strength, launching and capacity calculations. | (08) |

***********