## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C.Machines(4109)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the Emf equation for DC generator and state the factors on which Emf generated in dc generator depends. |  |
| b) | Discuss the various factors responsible for failure of DC generator to build up its voltage. |  |
| c) | Explain the principle of working of DC motor. |  |
| d) | Explain the process of positioning of brush in DC machine. |  |
| e) | Why does the field winding of dc series machine have less number of turns than that of dc shunt machine. |  |
| f) | What are the conditions to be fulfilled for parallel operation of DC generator and what are the advantages of connecting two or more DC generators in parallel? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the classification of DC generators. Explain each type in detail with neat sketch. Also write equation for terminal voltage and armature current for each type. |  |
| b) | List out different parts of DC machine and explain function of each part in detail. |  |
| c) | A six pole DC shunt generator with a lap connected armature having field and armature resistance of 100  and 0.02  respectively supplies 60 numbers of 200 volt, 60 watts lamp at rated voltage. Calculate - Total armature current - Current per armature path - Generated Emf allowing drop of 1 volts per brush. - Power loss in shunt field and armature. |  |
| d) | A 8 pole lap wound armature rotated at 350 rpm generates a voltage of 260 volts. The useful flux per is 0.05 wb. If the armature has 120 slots, calculate suitable number of conductors per slot and hence determine the actual value of flux required to generate the same voltage. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on current controller of DC welding generator. |  |
| b) | Explain how parallel operation of Dc compound generator is established. |  |
| c) | Two shunt generators operate in parallel and supply a common load of 800 Amps. Generator ‘1’ gives 240 volts at No load and 225 volts at 500 Amp output. Generator ‘2’ gives 245 volts at No load and 225 volts at 400 Amps. Assuming their load characteristics as linear. Determine the busbar voltage and Kw shared by each generator. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw and explain motor characteristics for series motor. Also give application of series motor. |  |
| b) | With a neat diagram explain construction and working of 3 point starter. |  |
| c) | A 10 HP, 230 volts shunt motor takes an armature current of 6 Amps from the supply at No load and runs at 1200 rpm, the armature resistance is 0.25 . Determine the speed and electro magnetic torque developed when the armature takes 36 Amps, the flux remaining constant. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain different types of DC motors and write the voltage equation in each case. |  |
| b) | Explain with a neat sketch Ward-Leonard method of speed control of DC shunt motor. Give its advantages and disadvantages. |  |
| c) | Explain the different speed control methods of Dc series motor and explain each in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on i) Inter poles ii) Compensating winding. |  |
| b) | Explain the commutation process in detail. |  |
| c) | The following figures gives the OCC of shunt generator at 300 rpm. Field Current (Amps); 0; 2; 3; 4; 5; 6; 7. Armature Voltage (Volts); 7.5; 92; 132; 162; 183; 190; 212. - Plot the OCC for 375 rmp and determine the voltage to which the machine will excite if field circuit resistance is 40 . - What additional resistance would have to be inserted in the field circuit to reduce the voltage to 200 volts at 375 rpm? |  |

***********