## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define a Flitched Beam and a Transformed section. |  |
| b) | Derive the expression for limit of eccentricity for a column of rectangular section for no tension condition. |  |
| c) | Define principal planes and principal stresses. |  |
| d) | State the assumptions made in theory of pure torsion. |  |
| e) | Draw influence diagram for Shear force, Bending moment and reactions for a simply supported beam. |  |
| f) | State the uses of Indeterminate structures. |  |
| g) | Define Middle Quarter Rule. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flitched beam consists of a wooden joist 150mm wide and 250mm deep strengthened by steel plates 10mm thick and 200mm deep on either side of joist symmetrically. If the stress in wood is 7 N/mm2, find the moment of resistance of beam. Take Es = 20Ew. |  |
| b) | A solid rod of bronze 50mm in diameter is surrounded by a fitting steel cylinder of external diameter of 80mm and internal diameter 50mm. If the section is subjected to Bending Moment of 500 Nm, determine stresses in steel and bronze. Take Es = 1.75Eb. |  |
| c)i) | Define Degree of Indeterminacy with examples. | (2) |
| ii) | Find degrees of Indeterminacy of following structures: - Propped cantilever - Two hinged arch - Fixed beam - Portal frame with both supports fixed. | (4) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular column 450mm wide, 600mm deep carries an axial load of 50KN. Another load of 250KN acts on YY axis at a distance of 100mm from C.G. of column. Determine maximum and minimum stresses in column. |  |
| b) | A Bridge column of 1500mm wide, 2000mm deep carries a vertical load of 2000KN at an eccentricity of 400mm above XX axis and 300mm to the right of YY axis of column. Determine the stresses at all corners of column. |  |
| c) | A circular rod of 80mm diameter is subjected to tension of 40KN which is off the axis of rod by 15mm. Determine maximum and minimum stresses in rod. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The normal stresses at a point in a beam material are 160N/mm2 tensile and 80N/mm2 compressive in two mutually perpendicular axis. Find normal, tangential and resultant stress on a plane inclined at 40º to the axis of major stress. |  |
| b) | Direct stresses of 120N/mm2 tensile and 90N/mm2 compressive exist on two perpendicular planes at a certain point in a body. They are also accompanied by shear stresses of 40N/mm2. Determine principal stresses in magnitude and direction and maximum shear stress in magnitude and direction. |  |
| c) | A screw jack shaft spindle is subjected to direct compressive stress of 80N/mm2 and shear stress of 35N/mm2 due to torque. Determine principal planes, principal stress and also maximum shear stress and its plane. Use Mohr’s circle method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported girder is of span 12m. A single rolling load of 40KN move on the girder. Determine maximum Shear Force and Bending Moment in girder along with location using ILD concept. |  |
| b) | A live load of 50 KN/m for a length of 4m is moving on the girder of span 16m which is simply supported. Determine Shear Force at a section of 6m from left. Also find maximum Shear Force in Girder. Use ILD for the same. |  |
| c) | A simply supported girder is of span 20m. A rolling UDL of 50 KN/m for a length of 5m is moving on it. Determine maximum Bending Moment at a section of 12m from left. Use ILD concept. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A hollow circular shaft of 250mm outside diameter, 200mm internal diameter transmits 80KW power at 150 rpm. Determine shear stress at the outer layer and inner layer of shaft. Maximum torque exceeds mean torque by 30%. |  |
| b) | A solid circular line shaft of 100mm diameter is subjected to Bending Moment 4000Nm and a torque of 6000Nm. Determine principal stresses and maximum shear stress in magnitude and direction produced in shaft. |  |
| c) | A solid circular line shaft rotates at 200 rpm transmits power of 75KW. Determine diameter of shaft if angle of twist not to exceed 1.6 degrees in a length of 2m shaft. Take rigidity modulus G = 80 KN/mm2. Maximum torque exceeds mean torque by 25%. |  |

***********