## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define density, specific gravity and specific weight. |  |
| b) | Give the relation between absolute pressure, gauge pressure and atmospheric pressure. |  |
| c) | Give a practical example of steady and uniform flow. |  |
| d) | With the help of a neat sketch explain vena contracta in case of flow through orifice. |  |
| e) | State the laws of fluid friction. |  |
| f) | State the conditions when a rectangular channel will be most economical channel. |  |
| g) | Define velocity of approach and give its significance in case of flow in open channels. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simple manometer (u) is used to measure the pressure of oil (sp.gr.=0.8) flowing in a pipe line. Its right limb is open to the atmosphere and left limb is connected to the pipe. The centre of the pipe is 9cm below the level of mercury (sp.gr. 13.6) in the right limb. If the difference of mercury level in the two limbs is 15 cms, determine the absolute pressure of the oil in the pipe in KN/m2. |  |
| b) | Explain the working of Bourdon Tube pressure gauge with the help of a neat sketch. |  |
| c) | A gate which is 2m wide and 1.2m high lies in a vertical plane and is hinged at the bottom. There is a liquid on the upstream side of the gate which extends 1.5 m above the top of the gate and has specific gravity of 1.45. On the down stream side of the gate there is water up to the top of the gate. Find i) the resultant force acting on the gate ii) the position of centre of pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following and give one practical example of each. - Uniform flow - Steady flow - Turbulent flow. |  |
| b) | The throat and exit diameters of convergent divergent mouthpiece are 5 cm and 10 cm respectively. It is fitted to a vertical side of a tank, containing water. Find the maximum head of water for steady flow. The maximum vacuum pressure is 8m of water and take atmospheric pressure equal to 76 cm of mercury. |  |
| c) | Derive the expression for the rate of flow of fluid through the venturimeter. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A tank has two identical orifices in one of its vertical sides. The upper orifice is 3m below the water surface and lower one is 5m below the water surface. The value of co-efficient of velocity Cv for each orifice is 0.96. Find the point of intersection of the two jets. |  |
| b) | A 30 metres long wein is divided into 10 equal bays by vertical posts, each 0.6m wide. Using Francis’s formula, calculate the discharge over the wein under an effective head of 1 metre. |  |
| c) | Derive formula for calculating loss of head due to sudden enlargement of pipe. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An open channel of most economical section, having the form of a half hexagon with horizontal bottom is required to give a maximum discharge of 20.2 m3/s of water. The slope of the channel bottom is 1 in 2500. Taking chezy’s constant, c=60 in chezy’s equation, determine the dimensions of the cross-section. |  |
| b) | A horizontal pipeline 40m long is connected to a water tank at one end and discharge freely into the atmosphere at the other end. For the first 25m of its length from the tank, the pipe is 150mm diameter and its diameter is suddenly enlarged to 300mm. The height of water level in the tank is 8m above the centre of the pipe. Considering all losses of head which occur determine the rate of flow. Take f=0.01 for both sections of the pipe. |  |
| c) | Find the bed slope of a trapezoidal channel of bed width 4 m, depth of water 3m and side slope of 2 horizontal to 3 vertical, when the discharge through the channel is 20 m3/s. Take Manning’s N= 0.03 in Manning’s formula C=1N m1/6. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of a hydraulic Ram. |  |
| b) | Distinguish clearly between centrifugal pump and reciprocating pump from the point of view of selection and principle of working. |  |
| c) | Write short notes on any two of the following: - Compressibility - Surface tension - Water hammer in pipes. |  |

***********