## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State Zeroth law of thermodynamics. |  |
| b) | A vacuum gauge of the condenser reads 640 mm of mercury? What is the absolute pressure in the condenser if the barometer reads 760 mm of mercury? |  |
| c) | Represent adiabatic process on P-V and T-S diagram. |  |
| d) | Define (i) Cut-off ratio (ii) Compression ratio in I.C engines. |  |
| e) | State Fourier Law of heat conduction. |  |
| f) | Distinguish between sensible heat and latent heat of steam. |  |
| g) | What are the requirements of a good refrigerant? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the Second law of thermodynamics as per (i) Kelvin-Plank (ii) Clausius. |  |
| b) | A Carnot engine works between temperature limits of 950°C and 175°C. The engine receives 4200 KJ of heat per minute. Determine the power of the engine and the amount of heat rejected to the sink per second. |  |
| c) | Explain why a gas has two specific heats. A quantity of a gas has volume of 0.5m3 at a pressure of 7 bar and a temperature of 110°C. If the temperature is increased to 260°C while the pressure remains constant, Calculate (i) The change in internal energy (ii) The external work done. Take Cp=1.005 KJ/KgK and Cv = 0.71 KJ/KgK. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | One Kg of air having an initial volume of 0.5 m3 is heated at constant pressure of 3.2 bar until the volume is doubled. Calculate, - Heat added (ii) Work done (iii) Initial and final temperature of air. Take Cp=1.005 KJ/KgK, R=0.29 KJ/KgK. |  |
| b) | A volume of 3 litre of a gas is allowed to expand from a pressure of 1500 KN/m2 absolute and temperature 1100°C to a final pressure of 300 KN/m2 absolute according to law PV1.28 = constant. Determine the final volume, temperature and work done by the gas during expansion. |  |
| c) | Represent Isothermal process on P-V and T-S diagrams. Also derive the relation for wok done, change in internal energy and heat transferred. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Plot Otto cycle on the temperature–entropy diagram. In an engine working on the ideal Otto cycle, the temperature at the beginning and the end of adiabatic compression are 120C and 503C respectively. Find the compression ratio and the air-standard efficiency of the engine. Take =1.4 for air. |  |
| b) | Explain with neat sketch working of two stroke cycle I.C. engine. |  |
| c) | A Diesel engine developing an indicated power of 40.5 KW consumes fuel of calorific value 45000 KJ/Kg at the rate of 9.8 Kg per hour. If 8 KW power is absorbed by friction and pumping losses within the engine, determine the indicated thermal efficiency and the brake thermal efficiency of the engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the conditions of steam in following cases. - At a pressure of 10 bar, total heat is 2646 KJ/Kg. - At a pressure of 15 bar, temperature is 197.39c. - At a pressure of 7 bar, specific volume is 0.25 m3. |  |
| b) | Find out total heat of evaporation and internal energy of 1Kg of steam at a pressure of 8 bar - When steam is wet having dryness fraction 0.9 - When steam is dry saturated - When the steam is superheated, the temperature of superheated steam being 210C. Assume specific heat of superheated steam as 2.26 KJ/KgK. |  |
| c) | The temperature inside and outside of a room are 20C and 32C respectively. Calculate the rate at which heat is being transferred through a glass window of area 1.5m2 and thickness 5 mm. Take K for glass as 0.84 w/mK. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a neat sketch of vapour compression refrigeration system and explain its working. |  |
| b) | The temperature limits of an ammonia refrigeration system are 25C and –10C. If the gas is dry at the end of compression, calculate the coefficient of performance of the cycle assuming no undercooling of the liquid ammonia. Use the following table for properties of ammonia. Temperature (C); Liquid Heat (KJ/Kg); Latent Heat (KJ/Kg); Liquid Entropy (KJ/Kg K). 25 -10; 298.9 135.37; 1166.94 1297.68; 1.1242 0.5443. |  |
| c) | (i) Define “Tonne of refrigeration”. |  |
|  | (ii)What are the advantages and disadvantages of a two stroke cycle engine over a four stroke cycle engine? |  |

***********