## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-II(4008)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw & explain various parts of a plain alidate. |  |
| b) | Derive relation between radius & degree of curve. |  |
| c) | What is contour gradient? Draw a contour representing a pond. |  |
| d) | What is a transition curve? |  |
| e) | What is the meaning of swinging the telescope & plunging the telescope? |  |
| f) | What is the use of an optical plummet? If due to some reason, it is not working then what would you do? |  |
| g) | What is latitude & departure of a line? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State different methods of plane table surveying. Explain the method you would use to locate an inaccessible point. |  |
| b) | What is orientation? State the methods of orientation & explain briefly. |  |
| c) | What is reconnaissance survey? What preliminary survey do you need for road construction? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any six characteristics of contours with neat sketches. |  |
| b) | Explain the graphical method of interpolating the contours. |  |
| c) | Explain briefly how you would measure the horizontal angle using a vernier theodolite. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | You are required to set a deflection angle of 30o 22’ 20”. Explain what procedure you would adopt to set out this angle. |  |
| b) | The measured lengths and bearings of the sides of a closed traverse ABCDE run in the anti clockwise direction & are tabulated below. Calculate lengths CD and DE. Line; Length in mtrs; Bearing. AB; 298.7; 0 0’. BC; 205.7; N 2512’ W. CD; ?; S 756’ W. DE; ?; S 5624’ E. EA; 213.4; N 3536’ E. |  |
| c) | State and explain different sources of errors in a theodolite. |  |

………2/-

-2-

| Q.No.5. | Answer any one of the following Questions: | (12) |
| --- | --- | --- |
| a) | Explain the field procedure for setting out a simple circular curve with Rankine’s method using two theodolites. |  |
| b) | Two tangents AB & BC intersect at point B at a chainage of 1200.15m calculate all the necessary data for setting out a curve of 100m radius & angle of intersection 150 by method of offset from long chord. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two distances of 50m & 80m were accurately measured & intercepts on staff between the outer stadia webs were 0.496 and 0.796 respectively. Calculate the tachometric constants. |  |
| b) | The following observations were taken with a tachometer fitted with an anallatic lens, the staff being held vertically. The constant of the tachometer is 100. Calculate the RL of B and distance between A & B. Instr. Stn; (Ht. of Inst) HOI; staff stn; Staff readings; Remarks. P; 1.255; BM; 1.325,1.825,2.325; RLof BM = 255.750 mtrs. P; 1.255; A; 0.850,1.600,2.350. B; 1.450; A; 1.715,2.315,2.915. |  |
| c) | State uses of contour. Define contour interval & horizontal equivalent. |  |

***********