## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List and describe in brief the various resistances to vehicle motion. |  |
| b) | Define overall transmission efficiency and obtain a relation for the same in terms of torque at the live wheels, engine torque and the various gear ratios. |  |
| c) | Describe rolling, pitching and bouncing motions in an automobile. |  |
| d) | Distinguish between sedan, hatchback and station wagon car body styles. |  |
| e) | State the condition for true rolling of the wheels during steering. |  |
| f) | Distinguish between fluid flywheel and torque convertor. |  |
| g) | List the two important parts of any suspension system and state the functions of each. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A motor car which weighs 18.14 KN including the four road wheels each of which has an effective diameter of 66 cm, a radius of gyration of 28 cm and a weight of 300N, has an engine developing 67.5 KW at 2400 r.p.m. The parts which rotate at engine speed weigh 1090N, with a radius of gyration of 11.5 cm. The transmission efficiency is 90% and the total road and air resistance at this engine speed in top gear of 3.84:1 is 890 N on level. Calculate the acceleration in m/sec2, under these conditions and assuming the acceleration to be uniform the time required to increase the speed by 32 Km/hr. |  |
| b) | A motor car with wheel base 275 cm with a centre of gravity 85cm above the ground and 115 cm behind the front axle has a coefficient of adhesion 0.6 between the tyre and the ground. Calculate the maximum possible acceleration when the vehicle, is - Driven on four wheels - Driven on the front wheels only - Driven on the rear wheels only. |  |
| c) | For typical motor car, the road resistance is given by 230N per tonne, the air resistance by the expression 0.0843 V2, transmission efficiency 88% in top speed, car weighs 2.032 tonnes when fully loaded. Calculate - The B.P. required for a top speed of 144 Km/hr - The acceleration in m/sec2 at 48 Km/hr, assuming the torque at 48 Km/hr in the top gear 25% more than at 144 Km/hr. - The B.P. required to drive the car up a gradient of 1 in 5 at 48 Km/hr, transmission efficiency 80% in bottom gear. The resistance being in N and V the speed in Km/hr and g=acceleration due to gravity=9.81 m/sec2. |  |

………2/-

-2-

| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a neat sketch, explain the working of a rack and pinion steering gear box. |  |
| b) | With the help of a neat sketch, explain the working of linkage type hydraulic power steering. |  |
| c) | A motor car has a wheel-base of 2.743m and pivot centre of 1.065 m. The front and rear wheel track is 1.217 m. Calculate the correct angle of outside lock and turning circle radius of the outer front and inner rear wheels when the angle of inside lock is 40. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a torque convertor. |  |
| b) | What is an overdrive? What are its advantages? |  |
| c) | With the help of a neat suitable sketch, explain the working of an automatic transmission. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a semi-elliptic leaf spring assembly with helper spring. Explain the purpose of the shackle. |  |
| b) | With the help of a neat labelled diagram, explain the construction and working of Mcpherson strut suspension. |  |
| c) | With the help of a neat sketch, explain the construction and working of a trailing link suspension. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the requirements of a good automobile body. |  |
| b) | Distinguish between integral and framed structure of an automobile. |  |
| c) | Explain how an automobile body is protected from rust. |  |

***********