## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain principles of survey. |  |
| b) | Draw a neat figure of 30m chain & explain its parts. |  |
| c) | Define offset & explain how it is taken in the field. |  |
| d) | Explain field procedure of chain & cross-staff survey. |  |
| e) | Explain local attraction & precautions to be taken during compass traverse to avoid local attraction. |  |
| f) | Define Bench mark & write down its types and where each one is preferred. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain indirect ranging method. |  |
| b) | Draw conventional signs for i) Embankment ii) Marshy land iii) Orchard iv) Quarry site. |  |
| c) | Explain chain & cross-staff survey. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention parts of prismatic compass & explain function of each. |  |
| b) | Explain graphical adjustment for closing error in case of compass traverse. |  |
| c) | A traverse ABCDA is made in the form of a square taking in clockwise order. If the bearing of AB is 115 30’ find the bearing of other sides. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following are the observed bearings of the lines of a traverse ABCDEA with a compass in a place where local attraction was suspected. Line; F.B.; B.B. AB; 191 45’; 13’ 0. BC; 39 30’; 222 30’. CD; 22 15’; 200 30’. DE; 242 45’; 62 45’. EA; 330 15’; 147 45’. Find the correct bearings of the lines. |  |
| b) | Mention the parts of tilting level and write down functions of each part. |  |
| c) | Explain temporary adjustments of Dumpy level. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain compound leveling. |  |
| b) | Write down the methods to calculate the reduced levels & explain any one. |  |
| c) | The following consecutive readings were taken with a level & & 4 metre leveling staff on a continuously sloping ground at common intervals of 30m. 0.935(on A), 1.540, 2.335, 3.015, 3.820, 0.315, 1.390, 2.05, 2.835, 3.425, 1.825, 1.935, 2.320, 3.025, 3.915(0n B) The R.L. of A was 19.40. Find the R.L. of all the points & apply the usual checks. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Ranging |  |
| b) | Profile levelling |  |
| c) | Planimeter |  |
| d) | Abney level |  |

***********