## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication of Boilers & Pressure Vessels(5341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the importance of IBR code |  |
| b) | Explain deformation and stress in flanges. |  |
| c) | What are wind and piping loads in the content of pressure vessels? |  |
| d) | What is an anchor bolt? |  |
| e) | Explain vertical plate bending machine. |  |
| f) | Explain plug and formed mandrels used in tube bending. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two boiler accessories with a neat sketch. |  |
| b) | Why there are discontinuity stresses in the vessel? How it is going to affect the pressure vessel? |  |
| c) | Explain the stress in bi-metallic joints. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various factors to be considered in selection of material for a pressure vessel? |  |
| b) | Explain with a neat sketch Pyramid type of a rolling machine. |  |
| c) | What is hot bending? Describe the method of lubrication for tube bending. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is spinning? How are spinning lathes different from ordinary machine shop lathe? |  |
| b) | What is stiffening? Explain any two methods of imparting stiffness to pressure vessels. |  |
| c) | Draw a neat stress-strain diagram and explain the following terms: i) Elastic limit ii) Ultimate strength iii) Toughness. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How stresses are analysed in thin cylindrical shell subjected to internal pressure? |  |
| b) | A spherical shell of 400mm diameter and thickness 10mm is filled with a fluid under pressure. Its volume is increased by 150cm3. Calculate the pressure exerted by the fluid on the shell. Take modulus of elasticity = 2 x 106 Kg/cm2 and poisson ratio = 0.3. |  |
| c) | What inspection is carried out during construction of a boiler prior to welding? |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Reinforced circular plates |  |
| b) | Design pressure and design temperature |  |
| c) | Hooks law |  |
| d) | Interpretation of welding radiographs |  |
| e) | Thermal stresses |  |

***********