## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(4346)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) Question No.1 is compulsory. Attempt any five

from the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | (15) |
| --- | --- | --- |
| a) | Static and dynamic stability |  |
| b) | Margin Line |  |
| c) | Tchebycheff’s bodyplan |  |
| d) | Centre of floatation |  |
| Q.No.2. | What are cross curves of stability? From the cross curves of stability, how are the G2 curves obtained for various displacements and loading conditions. | (12) |
| Q.No.3. | Box shaped vessel having length 75m, breadth 14 m and depth 4m floats on an even keel at a draft of 3.2m in sea water. Find the trim in the vessel if a forward compartment of 5m long is bilged. The compartment is next to the stem. | (12) |
| Q.No.4. | The following information has been obtained from the cross curves of stability of a ship of displacement 40,000 tonnes. Angle of Heel (); 0; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN (meters); 0; 0.9; 1.92; 3.11; 4.25; 6.3; 8.44; 9.39; 9.29; 8.5. | (12) |
|  | For KG=10m, determine a) Initial Metacentric Height (GM) b) Dynamic Stability at 15. |  |
| Q.No.5. | Answer the following: |  |
| a) | What are classification societies? Describe the role of such societies in respect of new construction. | (6) |
| b) | Describe the difference between stability at small angle of inclination and that at large angle of inclination. | (6) |
| Q.No.6. | Answer the following: |  |
| a) | Draw the typical view of transverse plain bulkhead. | (6) |
| b) | A ship of 10,000 tonnes displacement has the following righting levers when inclined. Heel (); 0; 10; 20; 30; 40; 50. GZ(m); 0; 0; 0.12; 0.21; 0.3; 0.3. | (6) |
|  | Calculate the dynamic stability. |  |
| Q.No.7. | Answer the following: |  |
| a) | Why G2=Gm sin cannot be used for large angle of stability? | (6) |
| b) | Explain stiff and tender ships. | (6) |

***********