## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mining Engg.

Subject: Mining Technology(4292)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Write short notes on a LOX cartridge. |  |
| 2) | Differentiate between direct and inverse initiation. |  |
| 3) | What are the problems associated with winzing? |  |
| 4) | How the ventilation arrangement is done in a sinking shaft? |  |
| 5) | Apart from instrumentation, what are the different ways of testing a roof? |  |
| 6) | List out the different steps involved in cyclic operations of a tunnel. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the salient features of a slurry explosive. |  |
| 2) | What is a magazine? Give the safety aspects that need to be considered in construction of a magazine. |  |
| 3) | Explain the construction of a delay detonator. What advantages does it offer compared to ordinary electric detonator? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | With a neat sketch explain the mechanics of rock blasting. |  |
| 2) | Explain the different types of controlled blasting. |  |
| 3) | 45 number of vertical holes of diameter 100mm are drilled on a coal bench of 6 mts height with a spacing & burden of 3.5 m and 3m respectively. A slurry explosive of 1.2 g/c.c density is charged in all the holes with required stemming. Find the powder factor of the explosives. Consider sub-grade drilling & assume other data if required. |  |
| Q.No.4. | Answer the following Questions: |  |
| 1) | Describe freezing method of shaft sinking with a neat sketch. | (8) |
| 2) | Draw a neat sketch of temporary lining of a sinking shaft & label it. | (4) |
|  | OR |  |
| 2) | Discuss full face method of tunnelling. | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the salient features of a hydraulic prop. |  |
| 2) | What are the types of roof bolts used in underground mines? Give the procedure of installing any one type of roof bolt. |  |
| 3) | With a neat sketch explain the two-compartment method of raising. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| 1) | What are the essential constituents of an explosive? |  |
| 2) | Write a short note on systematic support rules. |  |
| 3) | “Deck charging economises blasting in heterogenous rocks”- Justify the statement. |  |
| 4) | Give the important features of a walling scaffold. |  |

***********