## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Diesel Engine Operation & Maintenance(5045)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms ‘scavenging’ and ‘supercharging’. How do the two processes enhance the engine operation? |  |
| b) | List the various safety devices installed on the diesel engines and what is their primary functions. |  |
| c) | Name four important parameters recorded during engine operation. How do these readings benefit the engine supervisor? |  |
| d) | What do you mean by salvaging of engine components? Draw an engine valve and show various defects that might occur on it. |  |
| e) | Make a list of preventive maintenance activities in the cooling system of the diesel engine. |  |
| f) | Give a brief account of applications of diesel engine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the pressurized lubrication system of a diesel engine used to propel a barge. |  |
| b) | Explain with neat sketches the operational aspects of the fuel injection pump. |  |
| c) | With a simple sketch explain how cylinder lubrication is achieved on large 2-stroke diesel engines used in ocean going liners. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various activities involved prior to starting the diesel engine for the first time after installation. |  |
| b) | With a simple sketch explain any one type of diesel engine governor. |  |
| c) | With a neat sketch showing the valve clearance, give the step by step procedure how to carry out the valve clearance setting. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | ‘Engine does not start or stalls just after starting’. Prepare a trouble shooting chart providing the probable causes and corrective actions to be initiated. |  |
| b) | Define ‘Preventive Maintenance’. Prepare a preventive maintenance chart for a diesel engine used to run a gen-set. |  |
| c) | List down the various important tools used for repair and maintenance. Explain their specific use during servicing of the engine. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain with suitable sketch wherever required the step by step procedure to carry out the cylinder head and cylinder block assembly. |  |
| b) | What is ‘ring ridge’? List atleast five common defects on the cylinder liner and their causes. |  |
| c) | With a simple sketch explain the valve lapping. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Oil contamination |  |
| b) | Turbo charging advantages and limitations |  |
| c) | Spare parts and repair manuals |  |
| d) | Stages of combustion |  |

***********