## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | The block coefficient of fineness of displacement (cb) |  |
| b) | Archimedes Principle |  |
| c) | Simpson’s first rule |  |
| d) | 5,8,-1 rule |  |
| e) | Types of equilibrium |  |
| f) | Stiff and tender ships |  |
| g) | KN curve |  |
| h) | The angle of vanishing stability |  |
| i) | MCT 1cm or MCTC. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Write main features and functions of: i)General Cargo ship OR ii) Tanker. |  |
| b) | Find the righting levers for a ship; cross curves of stability (KN curves) for which are supplied in Fig.2 for various angles of heel, when the displacement is 35000 tonnes. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A ship of 6000 tonnes displacement has KG= 6m and KM=7.33m. The following cargo is loaded: 1000 tonnes, KG 2.5m 500 tonnes, KG 3.5m 750 tonnes, KG 9.0m The following cargo is then discharged: 450 tonnes, KG 0.6m 800 tonnes, KG 3.0m Find the final GM. |  |
| b) | Construct the curve of statical stability for the ship from the cross curves of stability (KN curves) for which are supplied in Fig.2, when the displacement is 35,000 tonnes and KG=9.0m. From the curves constructed, find the following: - the range of stability - the angle of vanishing stability - The maximum righting lever and the angle of heel at which it occurs - The approximate initial metacentric height. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain how lines are faired from given table of off-sets. | (08) |
| b) | Using the hydrostatic curve supplied in Fig.1, find out following information for draft 7.6m: i) TPC ii) MCT 1cm iii) Displacement iv) Longitudinal C.F. forward of amidships v) Longitudinal C.B. | (04) |

………2/-

-2-

| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A ship 75m long has semi-ordinates at the load water plane commencing from forward as follows: 0,1,2,4,5,5,5,4,3,2 and 0 meters respectively. The spacing between the first three semi-ordinates and the last three semi-ordinates is half of that between the other semi-ordinates. Find the position of the centre of floatation relative to amidships. |  |
| b) | A ship of 6000 tonnes displacement has KM=7.3m and KG=6.7m and is floating upright (No list). A weight of 60 tonnes already on board is shifted 12m transversely. Find the resultant list. |  |
| Q.No.6. | Answer the following Question: | (12) |
|  | A ship 100m long arrives in port with drafts 3m F and 4.3m A. TPC 10 tonnes, MCT 1cm 120 tonnes-m. The centre of floatation is 3m aft of amidships. If 80 tonnes of cargo is loaded in a position 24m forward of amidships and 40 tonnes of cargo is discharged from 12m aft of amidships, what are the new drafts? |  |