## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State advantages and disadvantages of AC transmission over DC transmission. |  |
| b) | State the factors to be taken into consideration while selecting size of distributor. |  |
| c) | Briefly explain what do you mean by ABC system for rural electrification. |  |
| d) | List the methods to improve earth resistance. |  |
| e) | Draw a neat circuit diagram for godown wiring. |  |
| f) | Mention any three important points about motor installation wiring. |  |
| g) | Where will you use following types of guy and why? (i) wind guy (ii) strut guy. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare radial, ring and interconnected system on the basis of load reliability of supply and economy. |  |
| b) | Explain SWER wire system with its advantages and disadvantages. |  |
| c) | Explain the various types of faults in distribution system and their effects on system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A single phase two wire feeder, 1500m long, supplies a load of 60A at 0.8 p.f., 40A at 0.85 p.f. and 50A at 0.88 p.f. at a distance of 600, 1200 and 1500m respectively from the feeding point. The resistance and reactance of the feeder per km length are 0.06 and 0.1 respectively. If voltage at far end is to be maintained at 220V, calculate the voltage at sending end. |  |
| b) | Explain the terms connected load, demand factor and diversity factor. |  |
| c) | List different components of distribution line and state function of each component. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Estimate the main material requirement for a 750m, 415/240V three phase four wire system in vertical configuration. The lines emanate from a substation to feed a load of 30 KW. Take span between two poles as 50m. The size of conductor is ACSR 6/1 x 2.59mm (code weasel) and weighs 128 Kg/Km. |  |
| b) | Discuss the advantages, disadvantages and applications of PVC casing capping wiring. |  |
| c) | Explain with a neat diagram plate earthing. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | State any six general rules for internal wiring. |  |
| b) | A hall is to be fitted with 10 lamps (100 watts) and 4 ceiling fans (60 watts). Dimension of the hall is 15m x 6m x 4.5m. - Draw a simple layout showing location of fans, tubes, distribution board and switch board. - Estimate the size and rating of cable, main board, distribution board and number of sub circuits required. |  |
| c) | Write a short note on MCB. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two ac 3 400V, 50Hz squirrel cage induction motors rated 15 Hp and 10Hp are to be installed in a workshop. Determine size of cables required, rating of main switch and motor switch required. Draw a single line diagram for the same. |  |
| b) | Give different methods for reducing earth resistance of a system. |  |
| c) | Draw a neat wiring diagram for subcircuit consisting of two lamps and 1 fan. |  |

***********