## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Production Engg.

Subject: Non Conventional Production Processes(5074)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the abrasives used in Abrasive Jet Machining. (AJM) |  |
| b) | State the advantages and limitations of Electrochemical Machining. (ECM) |  |
| c) | State various applications of chemical machining. (CHM) |  |
| d) | State the advantages and disadvantages of Electrical Discharge Machining. (EDM) |  |
| e) | State advantages and limitations of Laser Beam Machining. (LBM) |  |
| f) | State the applications of Electron Beam Machining. (EBM) |  |
| g) | Explain the principle of operation of Plasma arc welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give advantages and applications of Water Jet Machining |  |
| b) | With a neat sketch explain the basic principle of metal removal in Abrasive Jet Machining (AJM). State the factors that affect metal removal rate. |  |
| c) | Briefly explain with a sketch, working of a Ultrasound machining (USM) process showing important elements. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the applications of Ultrasound machining (USM)? Why can very hard material be cut better in USM than soft one. |  |
| b) | Discuss various factors which govern the accuracy of the parts produced by Electrochemical machining. (ECM) |  |
| c) | With the help of neat sketches discuss various methods of electrolyte flow in ECM. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe Electrochemical Grinding (ECG) process giving neat sketch. |  |
| b) | Explain the process of chemical milling giving its applications. |  |
| c) | Explain the basic principle of Electrical Discharge Machining (EDM) giving neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief different flushing techniques used in Electrical Discharge machining (EDM) giving their relative merits. |  |
| b) | Explain the setup for Electrical Discharge wire-cutting. |  |
| c) | Explain with a neat sketch principle of operation of Laser Beam Machining. (LBM) |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Electrical Discharge Grinding process |  |
| b) | Electron Beam Machining (EBM) |  |
| c) | Application of Plasma arc machining |  |
| d) | Explosive forming |  |