## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(S-4214)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Briefly explain direct and indirect calibration. |  |
| b) | Define Transducer and Inverse transducer. Give an example of each. |  |
| c) | Compare the performance of wire wound POT with non-wire wound POT. |  |
| d) | What is a dummy gauge? Why is it required? |  |
| e) | State two advantages and disadvantages of iron core coil over air core coil. |  |
| f) | Explain the principle of operation for a capacitive transducer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the basic blocks of a generalized instrumentation system? Explain the function of each block. |  |
| b) | What is the importance of “Standard” in Instrumentation? Classify and briefly explain various standards in Instrumentation. |  |
| c) | Describe the function of primary and secondary transducer using an example having bourdon tube and load cell. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a sketch explain 4 lead and 3 lead methods of compensation. |  |
| b) | State the principle of operation of a Thermistor. Describe the constructional details of a Thermistor and draw the following typical characteristics i) Temperature v/s Resistivity ii) Time v/s current. |  |
| c) | Describe the procedure of bonding resistance strain gauge. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the output voltage expression for a POT. Explain the input-output relationship of a POT. Also describe how sensitivity of POT can be improved. |  |
| b) | With diagram, briefly explain two non-self generating inductance transducers. |  |
| c) | Explain the use of LVDT for linear and rotary motion measurement. (use diagrams also) |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | With a sketch, explain the effect of production of eddy current change on the self inductance of a coil. |  |
| b) | Explain the operation of magnetostrictive transducer with the help of a diagram. |  |
| c) | How is a parallel plate capacitor used as a transducer? Explain with an example. |  |
| d) | Give 2 points of differentiation between: - Analog and digital Instruments - Power generating and Power operating. |  |

………2/-

-2-

| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a schematic diagram, explain the use of a capacitive transducer for measuring level of a non-conducting liquid. |  |
| b) | Draw and explain a capacitive transducer working on the principle of change in capacitance with change of area due to displacement. |  |

***********