## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1)  All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by moment of a force? Stating its units give mathematical expression. |  |
| b) | Giving an example, state principle of transmissibility. |  |
| c) | Explain the law of parallelogram of forces. |  |
| d) | For complete specification, what are the characteristics of a force? |  |
| e) | Giving an example for each type, state the classification of forces. |  |
| f) | What are the effects of torsion in a structure? Explain. |  |
| g) | Explain the effects of following on a structure: i) creep ii) settlement of foundation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of stress-strain diagram, explain the salient features of structural steel. |  |
| b) | What do you understand by “concrete mix design”? Explain various steps involved in it. |  |
| c) | Stating their properties list out different grades of concrete and steels used in various structures. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between: - One way and Two way slabs - Load bearing and framed structures. |  |
| b) | Describe the following: - Valutes - Domes. |  |
| c) | Describe special considerations needed for load bearing structures in seismic zones. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i)Find the centroid of a ‘T’ section having flange 200mmx12mm and web 150mmx10mm. ii)Find the centroid of an unequal angle section 75mm wide x 150mm deep x 8mm thick. |  |
| b) | Describe the relationship between intensity of load, shear force and bending moment. |  |
| c) | Sketch SFD and BMD for a cantilever having span ‘L’ subject to udl of ‘w’ throughout the span. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A simply supported beam 5m span subject to point loads 18KN, 27KN and 16KN at 1m, 2.5m and 3.5m from left support. Draw SFD and BMD. |  |
| b) | A cantilever beam 2.5m span carries an udl of 20KN/m throughout the span and a point load of 35KN at free end. Sketch SFD and BMD. |  |
| c) | A simply supported beam 4m, subject to uniformly varying load having maximum intensity of 20 KN/m at the centre of span and zero at left support. A point load of 30 KN acts at 3m from left support. Draw SFD and BMD. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Tensile and shear cracks |  |
| b) | Choice of materials and their specifications for restoration of structures |  |
| c) | Preventive measures needed in buildings to avoid occurrence of cracks |  |
| d) | Rehabilitation of structures |  |
| e) | Structural and Non-structural cracks |  |

***********