## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Food Technology

Subject: Food Engineering-II(4278)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| 1) | List the components of a Belt conveyor. |  |
| 2) | List the limitations of a screw conveyor in a food industry. |  |
| 3) | Name three food products that can be conveyed using pneumatic conveyor. |  |
| 4) | Give two major industrial applications of Bernoulli’s theorem. |  |
| 5) | Suggest suitable pumps to pump any two of the following products: (a) Milk (b) Mango Pulp (c) Orange Juice. |  |
| 6) | How does a Compressor differ from a Blower? |  |
| 7) | Name the filter aids used in food industry. |  |
| 8) | Name three graders used in fruit and vegetable industry. |  |
| 9) | Define Reverse osmosis. |  |
| 10) | State the role of baffles in mixing operations. |  |
| 11) | What type of equipment is used for obtaining: - Chilli powder from dry Red chillies - Wheat flour from wheat grains. |  |
| 12) | Suggest suitable filling machines for the following: - Milk - Potato chips. |  |
| Q.No.II. | Answer any four of the following Questions: | 4 x 3 = 12 |
| 1) | Draw a neat sketch of Pneumatic Conveying System and label the parts. |  |
| 2) | What is Reynold’s number? How is it used in determining the flow pattern in liquids? |  |
| 3) | Explain the working of a reciprocating compressor. |  |
| 4) | Explain with reason the filter you would choose for - Preparing lime juice cordial - To filter starch slurry. |  |
| 5) | Explain the working of a Basket press. |  |
| Q.No.III. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | State the advantages and disadvantages of pressing and solvent extraction for oil extraction. |  |
| 2) | Explain the working of Ribbon blender with a neat diagram. List at least two applications in food industry. |  |
| 3) | Explain the working of a planetary mixer and give its applications. |  |
| Q.No.IV. | Answer any three of the following Questions: | 3 x 4 = 12 |
| 1) | What is sieve analysis? How is it useful in food industry? |  |
| 2) | With a neat sketch explain the working of a fruit pulper. Give at least two applications. |  |
| 3) | Explain the working of a Pin mill and give its applications. |  |
| 4) | What is an ‘Emulsion’? Explain the role of homogenisers in stabilizing an emulsion. | ….2/- |

-2-

| Q.No.V. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| 1) | Explain with the help of diagrams, the type of currents generated in mixers used for liquids. Also state what aspect of design is related with these currents. |  |
| 2) | Explain the working of a Form-Fill-Seal machine. |  |
| 3) | Explain the working of fillers used for carbonated beverages. |  |
| Q.No.VI. | Write short notes on any three of the following: | 3 x 4 = 12 |
| 1) | Bowl centrifuge |  |
| 2) | Coffee extractor |  |
| 3) | Conchers |  |
| 4) | Pressure gauges |  |

***********